# VF ADAS Database

# Export of APA-V-Parking Comfort System Requirements

Produced by Dina Abdelmagid

Exported from HEAD

Sep 26 2025 04:39

## Table of contents

Please right-click and choose "Update Field" to initialize this field.

## APA-V-Parking Comfort System Requirements overview

System Requirements

## APA-V-Parking Comfort System Requirements

### 1 Revision History:

| CodeBeamer reference: | APAVPCSR-3415635 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

| Rev. No | Date | Author | Reviewed/ Approved by | Description |
| --- | --- | --- | --- | --- |
| 0.1 | 23-May-25 | Abirami K | - | Initial version of System requirements specification for V-Parking Comfort |
| 0.2 | 28-May-25 | Abirami K | Pallavi Advitot | Peer review fixes done across the document |
| 0.3 | 30-May-25 | Abirami K | Hamsa | SQA review comments fixed across the document |
| 1.0 | 30-May-25 | Abirami K | Josip F | Baselined |
| 1.1 | 21-June-25 | Abishna M P, Archana K, Neethu N K | - | Updated the existing requirements and added new requirements |
| 1.2 | 24-June-25 | Abishna M P, Archana K, Neethu N K | Sreejith Pai | Peer review fixes done across the document |
| 1.3 | 30-June-25 | Abishna M P, Archana K, Neethu N K | Hamsa T N | SQA review comments fixed across the document |

### 2 Introduction

| CodeBeamer reference: | APAVPCSR-3415636 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

### 3 Purpose

| CodeBeamer reference: | APAVPCSR-3415637 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 3.1 --

| CodeBeamer reference: | APAVPCSR-3415655 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

The purpose of this document is to clearly define the functional, non-functional and safety requirements of V-Parking Comfort feature for Vinfast automotive VF8, VF9 while taking customer needs and legal requirements into consideration.
This document provides developers with a detailed blueprint, facilitates clear communication among stakeholders, assists project managers in planning and managing the development process, serves as a reference for testers to ensure quality, and documents the requirements and decisions made during development for future reference and maintenance.

### 4 Scope

| CodeBeamer reference: | APAVPCSR-3415638 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 4.1 --

| CodeBeamer reference: | APAVPCSR-3415656 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

The scope of this document encompasses the detailed specifications and functional requirements necessary for the development and implementation of a reliable and efficient V-Parking Comfort Parking system. This document will outline V-Parking Comfort system’s objectives, including enhancing driver safety and minimizing the risk of collisions while parking. It will cover hardware and software requirements for real-time data processing and user interface design. Additionally, the document will address compliance with relevant industry standards and regulations.

### 5 Definitions Acronyms and abbreviations

| CodeBeamer reference: | APAVPCSR-3415639 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 5.1 --

| CodeBeamer reference: | APAVPCSR-3415657 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

ACM : Airbag Control Module
APA : Advanced Parking Assistance
APS : Advanced Parking System
BCM : Body Control Module
EPB : Emergency Park Brake
MHU : Multimedia Head Unit
IDB : Integrated Dynamic Breaking
IGN : Ignition
MHU: Multimedia Head Unit 
POV - Privately Owned Vehicle
SV : Subject Vehicle
SVM: Surround View Monitoring
TBD : To Be Decided
T-Box : Telematics Box
USS : Ultra Sonic Sensor
VCU : Vehicle Control Unit
: V-parking Comfort
XGW : Communication gateway

### 6 General Description

| CodeBeamer reference: | APAVPCSR-3415640 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

### 7 System Overview

| CodeBeamer reference: | APAVPCSR-3415641 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 7.1 --

| CodeBeamer reference: | APAVPCSR-3415658 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

### 8 Overview

| CodeBeamer reference: | APAVPCSR-3415642 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 8.1 --

| CodeBeamer reference: | APAVPCSR-3415659 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort in the Advanced Parking Assist System (APAS) is a cutting-edge feature developed to automate and streamline the parking experience while taking into account the driver’s parking slot preference. V-Parking Comfort helps drivers in locating and picking an optimal parking space, executing automatic parking maneuvers into the selected slot (Park in), and safely exiting a parking space (Park out), all under the supervision of the driver.

Park in: Utilizing camera and ultrasound sensors, the system offers a 3-D visualization of the available parking spots. Once the driver confirms their selected slot, the system autonomously maneuvers into the space with minimal effort from the driver. The user shall oversee the system continuously and can halt the vehicle by discontinuing input to the remote device. Ongoing input to the remote device (e. g. , touch, press, or a designated pattern) is necessary for the vehicle to persist in the parking maneuver. If the user withdraws their input, the vehicle will come to a halt. The user can Then either continue the parking maneuver or assume control of the vehicle.

Park out: The driver can initiate V-Comfort park out via mobile device or key fob. Once activated, the VPC system exits the vehicle from the parking space.

#### 8.2 --

| CodeBeamer reference: | APAVPCSR-3415660 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

Current vehicle Inputs:
Vehicle speed : Current vehicle current speed
Vehicle longitudinal acceleration : Current vehicle current longitudinal acceleration
Vehicle lateral acceleration: Current vehicle current lateral acceleration
Pitch angle : Current vehicle pitch angle
Acceleration pedal position : Position of the acceleration pedal of the Current vehicle
Braking applied by driver Pressure applied by the driver to the braking system
Steering angle: Current vehicle steering wheel angle
Gear position: Selected gear of the Current vehicle
Power status : Current vehicle power status (powered on or powered off)
Door’s status : Open or close for driver and passengers’ doors
Trunk status : Current trunk status (opened, closed...)
Hood status : Current hood status (opened, closed...)
Mirrors : Current hood status (Folded, Un-folded...)
Battery : State of Charge, 
Turn indicator: on/ off, 
Remote status : remote device connection status
AEB status : Current status of the AEB feature
FCTA status (FCW) : Current status of the FCTA feature
RCTA status (RCW) : Current status of the RCTA feature
ACC status : Current status of the ACC feature Related system failure Fault in the ESC, ABS, EPS, EPB, VCU, TCU(T-BOX, MHU, encryption error, sensor error, engine failure, low Tyre pressure TPMS.

#### 8.3 --

| CodeBeamer reference: | APAVPCSR-3415661 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

Environment (Outside of the vehicle Inputs) :
Object type : List containing the type of object of each detected object
Object head : List containing the orientation of each detected object
Object x position : List containing the longitudinal coordinate of each detected object
Object y position : List containing the transversal coordinate of each detected object
Object velocity: List containing the relative velocity in the heading direction of each detected object (ego car health)
Object acceleration : List containing the acceleration of each detected object
Object width : List containing the width of each detected object
Object length : List containing the length of each detected object
Lines information : Information for the right and left lines (distance, line coefficients, quality, type, validity range, time to line crossing)
Road surface conditions : Whether the roads are ODD conditions or not need to refer the ODD section
Road type : Current road type
Weather status : Current weather status (raining, cloudy, snow, sunny...)
Visibility status : Current visibility status (good, moderate, poor...) or value of the visual distance (meters)
Lighting status : Current illuminance status (good, moderate, poor...) or value of the illuminance (lux)

#### 8.4 --

| CodeBeamer reference: | APAVPCSR-3415662 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

MHU Inputs:
Activated / Not activated : Activated by user button press 
RP selection : (V-parking Comfort) Through C-app or Key fob Remote parking option selection.

#### 8.5 --

| CodeBeamer reference: | APAVPCSR-3415663 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

Remote C-App / Key Fob Input:
Start confirmation: Button press to start Maneuvering 
Parking Slot Confirmation : Confirming of suggested parking slot
Vehicle orientation : View the vehicle orientation on screen
Remote controller status: Current remote controller status (remote connection quality...) Syncing or pair
Remote control status : Current remote-control status or driver actions on the remote controller (continuous action , suspend, resume, cancel)
Remote controller position : Current remote controller position (outside the car in a key fob or c-app within the desired range as per the Market EU, US, VIETNAM) 6 to 30m) [TBD]
Battery level: Battery level of Key fob or Mobile

#### 8.6 --

| CodeBeamer reference: | APAVPCSR-3415664 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

Current Vehicle outputs:
RP (V-parking Comfort status): Current feature status (disabled, failure, standby, maneuvering...)
Requested acceleration/deceleration: Current acceleration request
Requested steering angle : Current steering angle request
Requested gear shift position : Current gear shift position request
Requested electronic parking brake Current electronic parking brake request
Requested right turn light : Current right turn light request
Requested left turn light : Current left turn light request
Requested emergency lights: Current emergency lights request
Requested power off : Current power off request to IGN OFF

#### 8.7 --

| CodeBeamer reference: | APAVPCSR-3415665 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

MHU output:
Screen output Messages are happened based on the request given the MHU Driver: TBD.
Notifications: Preconditions of current vehicle data notifying

#### 8.8 --

| CodeBeamer reference: | APAVPCSR-3415666 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

Remote C-App/Key Fob Output:
Screen output Messages are happened based on the request given by the Driver: TBD.
Slot detected: Based on the Outside info given the information 
Path maneuvering: Planned Trajectory 
Notifications: Preconditions of current vehicle data notifying
Safety Messages : Message on Driver Override request
SVM Status : Images of SVM and outside monitoring system
Current Maneuver information and status: Percentage of path covered, expected completion time
Park in /park-out status : Through the C-app notify to use the if parking is completed successfully.

### 9 External Interface with Other Devices

| CodeBeamer reference: | APAVPCSR-3415643 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 9.1 --

| CodeBeamer reference: | APAVPCSR-3415667 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

1. Key-fob
2. C-APP

#### 9.2 --

| CodeBeamer reference: | APAVPCSR-3415668 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

#### 9.3 Communication Interface Requirements

| CodeBeamer reference: | APAVPCSR-3415669 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 9.4 --

| CodeBeamer reference: | APAVPCSR-3415670 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

TBD (CAN DBC MATRIX NEED TO BE PROVIDED BY THE VINFAST TEAM)

### 10 User Characteristics

| CodeBeamer reference: | APAVPCSR-3415644 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 10.1 --

| CodeBeamer reference: | APAVPCSR-3415671 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort feature of Advanced Parking Assist (APA) system shall be designed to accommodate the diverse characteristics and needs of its users, ensuring a seamless and intuitive parking experience. It shall be intuitive and easy to use for drivers of all ages, including young adults and elderly users, and support multiple languages. The system shall provide a user-friendly interface accessible to both novice and experienced driver, featuring clear and simple instructions. shall provide information to the user via visual, auditory AND HAPTIC feedbacks. The system shall enable users to park their vehicles accurately and safely with minimal effort, supporting inside operation and remote operation from outside the vehicle. It shall function effectively in various parking environments and weather conditions, and include features to assist users with disabilities, such as voice commands and large, easy-to-read display options. The system shall comply with accessibility standards and recognize common user behaviors, providing quick and efficient parking solutions to accommodate users with busy schedules and time constraints.

### 11 General Constraints

| CodeBeamer reference: | APAVPCSR-3415645 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 11.1 --

| CodeBeamer reference: | APAVPCSR-3415672 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

1. V-Parking Comfort System can operate in Good and moderate illumination with Good and Moderate Visibility(during rainfall / snowfall / fog / smoke / smog / sleet) of 75 to more than 300mts.
2. V-Parking Comfort System shall Operate in ambient temperatures between less than -20 to 45 °C .

### 12 General Assumptions

| CodeBeamer reference: | APAVPCSR-3415646 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 12.1 --

| CodeBeamer reference: | APAVPCSR-3415673 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall provide a visual and audio warning message if key fob is used for driver communication ([Red LED illuminates Continually) to inform the driver that the subject vehicle has been temporarily stopped due to the presence of an obstacle in the trajectory while the obstacle is present in the trajectory and the feature is in MANOEUVRING state.

#### 12.2 --

| CodeBeamer reference: | APAVPCSR-3415674 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

The V-Parking Comfort shall provide a visual and audio warning message eg: Red LED illuminates for 4 seconds tunable and Then IGN turns off to inform the driver if the driver use only key fob for communication that the maneuver has been suspended , When the feature transitions from MANUEVRING to SUSPENDED state.

### 13 Variants to be offered

| CodeBeamer reference: | APAVPCSR-3415647 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 13.1 --

| CodeBeamer reference: | APAVPCSR-3415675 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

No Feature Variants

### 14 User Interface

| CodeBeamer reference: | APAVPCSR-3415648 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

### 15 Screen Formats

| CodeBeamer reference: | APAVPCSR-3415649 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 15.1 --

| CodeBeamer reference: | APAVPCSR-3415676 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

NA

### 16 Warnings:

| CodeBeamer reference: | APAVPCSR-3415650 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 16.1 --

| CodeBeamer reference: | APAVPCSR-3415677 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering 
AND
VPC_Paused is True
Then
The V-Parking comfort shall set Reason_for_suspend as Req_Recover_Press_Pause(8)

#### 16.2 --

| CodeBeamer reference: | APAVPCSR-3415678 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When 
VPC_Status is Path Maneuvering
AND
IDB_Brake_Override_Level is Hard brake
Then 
The V-Parking comfort shall set Reason_for_suspend as Req_Recover_Press_brake (4).

#### 16.3 --

| CodeBeamer reference: | APAVPCSR-3415679 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When 
VPC_Status is Path Maneuvering
AND
Obstcl_Identified is True
AND
Alternate_Path_Planning is False 
Then 
The V-Parking comfort shall set Reason_for_suspend as Req_Recover_FoundObstacleInTrajectory (2)

#### 16.4 --

| CodeBeamer reference: | APAVPCSR-3415680 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path Maneuvering 
AND
ACM_Seat_Belt_Status_Driver is unbuckled
Then
The V-Parking comfort shall set Reason_for_suspend as Req_Recover_driver_seatbelt_unbuckle (5)

#### 16.5 --

| CodeBeamer reference: | APAVPCSR-3415681 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

When
VPC_Status is Path Maneuvering
AND
BCM_STAT_TrunkAjar is open
Then
The V-Parking comfort shall set Reason_for_suspend as Req_Recover_Luggage Door Open (3)

#### 16.6 --

| CodeBeamer reference: | APAVPCSR-3415682 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

When 
VPC_Status is Path Maneuvering
AND
BCM_Doors_Lock_Status is Unlocked 
Then
The V-Parking comfort shall set Reason_for_suspend as Req_Recover_Doors_unlock (1)

#### 16.7 --

| CodeBeamer reference: | APAVPCSR-3415683 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When
VPC_Status is Path maneuvering
AND
BCM_STAT_Mirror is folded
Then
The V-Parking comfort shall set Reason_for_suspend as Req_Recover_Fold_Mirror (10)

#### 16.8 --

| CodeBeamer reference: | APAVPCSR-3415684 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

When
VPC_Status is Path Maneuvering 
AND
object remains in parking path more than MAX_OBSTACLE_WAIT_TIME ( predefined threshold time) 
Then
The V-Parking Comfort shall set Reason_for_abort as object remains in parking path more than predefined threshold time(37)

#### 16.9 --

| CodeBeamer reference: | APAVPCSR-3415685 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When
VPC_Status is Path Maneuvering
AND
C-App_Maneuver_Command is False
OR
Keyfob_Maneuver_Command is False
Then
The V-Parking comfort shall set Reason_for_abort as driver stops engagement through the remote device(1)

#### 16.10 --

| CodeBeamer reference: | APAVPCSR-3415686 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |
| AVPCSR-3417045 | -- |

No Downstream References

No Associations

Description

When
VPC_Status is Path Maneuvering 
AND
Manual_override is set as True / EPS_Steering_Control_Status is Manual Override
OR
EPB_Control_Status is Manual Override
OR 
VCU_Gear_Control_Status is Manual Override
OR
IDB_Brake_Override_Level is Hard brake
OR
VCU_Throttle_Control_Status is Manual Override
Then
The V-Parking comfort shall send Reason_for_abort as Manually Overrided wrt steering, acceleration or brake(2)

#### 16.11 --

| CodeBeamer reference: | APAVPCSR-3415687 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

When
VPC_Status is Path Maneuvering 
AND
Sensor_Malfunction is True
Then
The V-Parking comfort shall set Reason_for_abort as Sensor_malfuction is True(3)

#### 16.12 --

| CodeBeamer reference: | APAVPCSR-3415688 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path Maneuvering 
AND
VPC_Fault_Status is True
Then
The V-Parking Comfort shall set Reason_for_abort as software error(4)

#### 16.13 --

| CodeBeamer reference: | APAVPCSR-3415689 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

When
VPC_Status is Path Maneuvering 
AND
Camera_Visibility_Status is Poor visibility Or Blocked
Then
The V-Parking comfort shall set Reason_for_abort as Poor visibility(5)

#### 16.14 --

| CodeBeamer reference: | APAVPCSR-3415690 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When 
VPC_Status is Slot Display and auto-maneuvering
AND
Distance reached is more than the MAX_DIST , Odometer_DistanceTrav is greater than the MAX_DIST
Then
The V-Parking comfort shall set Reason_for_abort as Parking slot unavailable

#### 16.15 --

| CodeBeamer reference: | APAVPCSR-3415691 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path Maneuvering 
AND
Number_of_moves/ Number of attempts to park-in or park-out exceed MAX_ATTEMPT_COUNT
Then
The V-Parking comfort shall set Reason_for_abort as maximum number of moves in a maneuver is exceeded(7)

#### 16.16 --

| CodeBeamer reference: | APAVPCSR-3415692 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

When
VPC_Status is Path Maneuvering 
AND
Low_battery_conditions is set True/ BMS_Battery_SoC < Battery_SoC_THD
Then
The V-Parking comfort shall set Reason_for_abort as low battery conditions are detected (8)

#### 16.17 --

| CodeBeamer reference: | APAVPCSR-3415693 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path Maneuvering 
AND
Mechanical/Electronic systems failure
Then
The V-Parking comfort shall set Reason_for_abort as Failure of mechanical/electronic systems (9)

#### 16.18 --

| CodeBeamer reference: | APAVPCSR-3415694 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path Maneuvering 
AND
Vehicle_speed (VCU_Veh_Spd) is greater than SLOT_SCAN_SPEED(maximum threshold vehicle speed )
Then
V-Parking comfort shall set as Reason_for_abort as Req_Quit_Speed_High(10)

#### 16.19 --

| CodeBeamer reference: | APAVPCSR-3415695 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path Maneuvering
AND
Improper calibration is True
Then
The V-Parking comfort shall set Reason_for_abort as Failure of Improper calibration (11)

#### 16.20 --

| CodeBeamer reference: | APAVPCSR-3415696 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path Maneuvering 
AND
Park-in or Park-out process exceeds MAX_MANUEVER_COMPLT_TIME
Then
The V-Parking comfort shall set Reason_for_abort as parking process exceeds the threshold time (12)

#### 16.21 --

| CodeBeamer reference: | APAVPCSR-3415697 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
EPB_Brake_State is set True
Then
V-Parking comfort shall set as Reason_for_abort as Req_Quit_EPB_Brake_State (13)

#### 16.22 --

| CodeBeamer reference: | APAVPCSR-3415698 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
Driver_Gear_Intervention is set True
Then
The V-Parking comfort shall set as Reason_for_abort as Req_Quit_Driver_Gear_Intervention(14)

#### 16.23 --

| CodeBeamer reference: | APAVPCSR-3415699 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (5)

| AVPCSR-3416599 | -- |
| --- | --- |
| AVPCSR-3416576 | -- |
| AVPCSR-3416577 | -- |
| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| AVPCSR-3417011 | -- |

No Downstream References

No Associations

Description

When
VPC_Status is Path Maneuvering 
AND
APA_Error_Status is set True
Then
The V-Parking comfort shall set Reason_for_abort as APA_system_failure (15)

#### 16.24 --

| CodeBeamer reference: | APAVPCSR-3415700 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

When
VPC_Status is Path Maneuvering 
AND
VPC_C-App_Communication_Status is Failed
OR
VPC_Keyfob_Communication_Status is Failed
Then
The V-Parking comfort shall set Reason_for_abort Remote_connection_timeout (16)

#### 16.25 --

| CodeBeamer reference: | APAVPCSR-3415701 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path Maneuvering 
AND
C-App_Maneuver_Command is False for more than MAX_TIME_SUSPEND
OR
Keyfob_Maneuver_Command is False for more than MAX_TIME_SUSPEND
Then
The V-Parking comfort shall set Reason_for_abort as Req_Quit_Pause_Timeout (17)

#### 16.26 --

| CodeBeamer reference: | APAVPCSR-3415702 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path Maneuvering 
AND
VCU_Throttle_Control_Status is Manual Override
Then
The V-Parking comfort shall set Reason_for_abort as Req_Quit_ACC (18)

#### 16.27 --

| CodeBeamer reference: | APAVPCSR-3415703 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path Maneuvering 
AND
HWA_requested is set True
Then
The V-Parking comfort shall set Reason_for_abort as Req_Quit_HWA (19)

#### 16.28 --

| CodeBeamer reference: | APAVPCSR-3415704 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (1)

| APASAD-3412216 | Sequence Diagram |
| --- | --- |

No Associations

Description

When
VPC_Status is Path Maneuvering
AND
MHU_VPC_Cancel_Req is True 
OR
Keyfob_VPC_Cancel_Req is True 
OR 
C-App_VPC_Cancel_Req is True
Then
The V-Parking comfort shall send Reason_for_abort as Req_Quit_Deactivation_Button_Pressed (20)

#### 16.29 --

| CodeBeamer reference: | APAVPCSR-3415705 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path Maneuvering
AND
Vehicle_blocked is set True
Then
The V-Parking comfort shall set as Reason_for_abort as Req_Quit_Vehicle Block (21)

#### 16.30 --

| CodeBeamer reference: | APAVPCSR-3415706 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |
| AVPCSR-3416577 | -- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
PAS_internal_failure is set True
Then
The V-Parking shall set as Reason_for_abort as Req_Quit_PAS_Failure (22)

#### 16.31 --

| CodeBeamer reference: | APAVPCSR-3415707 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
External_ECU_failure is set True
Then
The V-Parking comfort shall send Reason_for_abort as Req_Quit_External_ECU_failure (23)

#### 16.32 --

| CodeBeamer reference: | APAVPCSR-3415708 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| AVPCSR-3417044 | -- |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path Maneuvering 
AND
VPC_C-App_Communication_Status is Failed 
Then
The V-Parking comfort shall send Reason_for_abort as Req_Quit_RD_Offline (24)

#### 16.33 --

| CodeBeamer reference: | APAVPCSR-3415709 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
RPA_Page_Off is set True
Then
The V-Parking comfort shall send Reason_for_abort as Req_Quit_RPA_Page_Off (25)

#### 16.34 --

| CodeBeamer reference: | APAVPCSR-3415710 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
RD_Failure is set True
Then
The V-Parking comfort shall send Reason_for_abort as Req_Quit_RD_Failure(26)

#### 16.35 --

| CodeBeamer reference: | APAVPCSR-3415711 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (1)

| APASAD-3412216 | Sequence Diagram |
| --- | --- |

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
RD_Deactivation_Request is set True
Then
The V-Parking comfort shall set as Reason_for_abort as Req_Quit_RD_Deactivation_Request(27)

#### 16.36 --

| CodeBeamer reference: | APAVPCSR-3415712 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
Moves_Times_overflow is set True
Then
The V-Parking comfort shall set as Reason_for_abort as Req_Quit_Move_times_overflow (38)

#### 16.37 --

| CodeBeamer reference: | APAVPCSR-3415713 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
Interrupt_Times_overflow is set as True
Then
The V-Parking comfort shall send Reason_for_abort as Req_Quit_Recover_Interrupt_Times_Overflow(28)

#### 16.38 --

| CodeBeamer reference: | APAVPCSR-3415714 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
Moving_Control_Timeout is set True
Then
The V-Parking comfort shall send Reason_for_abort as Req_Quit_MovingControl_Timeout (29)

#### 16.39 --

| CodeBeamer reference: | APAVPCSR-3415715 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path Maneuvering 
AND
Unsafe_behavior_detected is set true
Then
The V-Parking comfort shall send Reason_for_abort as Req_Quit_Unsafe_Behavior (30)

#### 16.40 --

| CodeBeamer reference: | APAVPCSR-3415716 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering 
AND
Slope_exceeds_limitation is set True
Then
The V-Parking comfort shall send Reason_for_abort as Req_Quit_Slope_exceed_limitation (31)

### 17 Error Messages

| CodeBeamer reference: | APAVPCSR-3415651 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 17.1 --

| CodeBeamer reference: | APAVPCSR-3415717 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

V-parking Comfort shall inform driver with error messages on the reason for suspension, reason for abort, handover the control to driver, misaligned parking slot (error parking slot) , parking slot or area full in the C-app.

### 18 Functional Requirements

| CodeBeamer reference: | APAVPCSR-3415652 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 18.1 Input /Output Processing

| CodeBeamer reference: | APAVPCSR-3415718 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.1.1 Input Signals

| CodeBeamer reference: | APAVPCSR-3415782 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

###### 18.1.1.1 --

| CodeBeamer reference: | APAVPCSR-3416102 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415501 | Actuator Command Transmission and System Responsiveness |
| --- | --- |

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal MHU_APA_Req from MHU.

###### 18.1.1.2 --

| CodeBeamer reference: | APAVPCSR-3416103 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |
| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |

No Downstream References

No Associations

Description

V-parking Comfort shall receive the signal EPB_Brake_State from EPB.

###### 18.1.1.3 --

| CodeBeamer reference: | APAVPCSR-3416104 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

V-parking Comfort shall receive the signal BCM_STAT_Mirror from CGW.

###### 18.1.1.4 --

| CodeBeamer reference: | APAVPCSR-3416105 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

V-parking Comfort shall receive the signal BCM_STAT_DoorAjarFL from CGW.

###### 18.1.1.5 --

| CodeBeamer reference: | APAVPCSR-3416106 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

V-parking Comfort shall receive the signal BCM_STAT_DoorAjarFR from CGW.

###### 18.1.1.6 --

| CodeBeamer reference: | APAVPCSR-3416107 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

V-parking Comfort shall receive the signal BCM_STAT_DoorAjarRL from CGW.

###### 18.1.1.7 --

| CodeBeamer reference: | APAVPCSR-3416108 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

V-parking Comfort shall receive the signal BCM_STAT_DoorAjarRR from CGW.

###### 18.1.1.8 --

| CodeBeamer reference: | APAVPCSR-3416109 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

V-parking Comfort shall receive the signal BCM_STAT_TrunkAjar from CGW.

###### 18.1.1.9 --

| CodeBeamer reference: | APAVPCSR-3416110 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

V-parking Comfort shall receive the signal ACM_Seat_Belt_Status_Driver from EPS.

###### 18.1.1.10 --

| CodeBeamer reference: | APAVPCSR-3416111 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415584 | Park-In Maneuver Initiation: Control mode change |
| --- | --- |

No Downstream References

No Associations

Description

V-parking Comfort shall receive the signal Keyfob_Maneuver_Command from CGW.

###### 18.1.1.11 --

| CodeBeamer reference: | APAVPCSR-3416112 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415584 | Park-In Maneuver Initiation: Control mode change |
| --- | --- |

No Downstream References

No Associations

Description

V-parking Comfort shall receive the signal C-App_Maneuver_Command from TCU.

###### 18.1.1.12 --

| CodeBeamer reference: | APAVPCSR-3416113 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415501 | Actuator Command Transmission and System Responsiveness |
| --- | --- |

No Downstream References

No Associations

Description

V-parking Comfort shall receive the signal Sensor_input_Status.

###### 18.1.1.13 --

| CodeBeamer reference: | APAVPCSR-3416114 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415501 | Actuator Command Transmission and System Responsiveness |
| --- | --- |

No Downstream References

No Associations

Description

V-parking Comfort shall receive the signal Camera_input_status.

###### 18.1.1.14 --

| CodeBeamer reference: | APAVPCSR-3416115 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415571 | Parking Feasibility Check |
| --- | --- |
| APAVPC-3415565 | Remote Mode Selection: Continuous Engagement via Mobile App |

No Downstream References

No Associations

Description

V-parking Comfort shall receive the signal Slot_markings_identified.

###### 18.1.1.15 --

| CodeBeamer reference: | APAVPCSR-3416116 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415571 | Parking Feasibility Check |
| --- | --- |
| APAVPC-3415565 | Remote Mode Selection: Continuous Engagement via Mobile App |

No Downstream References

No Associations

Description

V-parking Comfort shall receive the signal MHU_Selected_Slot_ID from MHU.

###### 18.1.1.16 --

| CodeBeamer reference: | APAVPCSR-3416117 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415542 | Path and Trajectory Planning: Traffic circulation |
| --- | --- |

No Downstream References

No Associations

Description

V-parking Comfort shall receive the signal VPC_button from MHU.

###### 18.1.1.17 --

| CodeBeamer reference: | APAVPCSR-3416118 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (3)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |
| APAVPC-3415564 | Low-speed forward navigation Execution |
| APAVPC-3415562 | Mid-maneuver Condition Check and Status Communication |

No Downstream References

No Associations

Description

V-parking Comfort shall receive the signal Obstcl_Identified.

###### 18.1.1.18 --

| CodeBeamer reference: | APAVPCSR-3416119 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415577 | Remote Mode Selection: Driver exit without Park brake |
| --- | --- |

No Downstream References

No Associations

Description

V-parking Comfort shall receive the signal C-App_Selected_ Slot_ID from TCU.

###### 18.1.1.19 --

| CodeBeamer reference: | APAVPCSR-3416120 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

V-parking Comfort shall receive the signal MHU_VPC_Cancel_Req

###### 18.1.1.20 --

| CodeBeamer reference: | APAVPCSR-3416121 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

V-parking Comfort shall receive the signal BCM_STAT_BonnetAjar.

###### 18.1.1.21 --

| CodeBeamer reference: | APAVPCSR-3416122 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

V-parking Comfort shall receive the signal IDB_BrakePedal_State from IDB.

###### 18.1.1.22 --

| CodeBeamer reference: | APAVPCSR-3416123 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (16)

| APAVPC-3415575 | Reserved Slot Display |
| --- | --- |
| APAVPC-3415581 | Persistent Obstacle: Abort maneuver by System |
| APAVPC-3415613 | Reuse |
| APAVPC-3415611 | Park-Out Cancellation and Safe Maneuver Termination |
| APAVPC-3415598 | Low-speed navigation: Initiation via MHU |
| APAVPC-3415590 | Park-In Maneuver terminated by Driver seated |
| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| APAVPC-3415621 | Support OTA Updates |
| APAVPC-3415600 | V_Comfort Park-Out Maneuver Controls |
| APAVPC-3415583 | Display Park-In Trajectory and progress |
| APAVPC-3415584 | Park-In Maneuver Initiation: Control mode change |
| APAVPC-3415564 | Low-speed forward navigation Execution |
| APAVPC-3415559 | Pre-Maneuver Condition Check and Status Communication |
| APAVPC-3415544 | Communicate activation status to driver |
| APAVPC-3415542 | Path and Trajectory Planning: Traffic circulation |

No Downstream References

No Associations

Description

V-parking Comfort shall receive the signal VCU_Gear_Ctrl_Status from VCU.

###### 18.1.1.23 VF_PF_SYSRS_1536

| CodeBeamer reference: | APAVPCSR-3416124 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal EPB_Ctrl_Status from EPB

###### 18.1.1.24 VF_PF_SYSRS_1537

| CodeBeamer reference: | APAVPCSR-3416125 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal VCU_Throttle_Ctrl_Status from VCU

###### 18.1.1.25 VF_PF_SYSRS_1538

| CodeBeamer reference: | APAVPCSR-3416126 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal IDB_Ctrl_Status from IDB

###### 18.1.1.26 VF_PF_SYSRS_1539

| CodeBeamer reference: | APAVPCSR-3416127 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal VCU_Veh_Spd from VCU.

###### 18.1.1.27 VF_PF_SYSRS_1540

| CodeBeamer reference: | APAVPCSR-3416128 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal VCU_Ign_On from VCU.

###### 18.1.1.28 VF_PF_SYSRS_1541

| CodeBeamer reference: | APAVPCSR-3416129 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal MHU_Selected_Park-out_Dir from MHU.

###### 18.1.1.29 VF_PF_SYSRS_1542

| CodeBeamer reference: | APAVPCSR-3416130 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal C-App_Selected_Park-out_Dir from TCU

###### 18.1.1.30 VF_PF_SYSRS_1543

| CodeBeamer reference: | APAVPCSR-3416131 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal MHU_User_Comm_dev from MHU

###### 18.1.1.31 VF_PF_SYSRS_1544

| CodeBeamer reference: | APAVPCSR-3416132 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal MHU_Maneuver_Command from MHU

###### 18.1.1.32 VF_PF_SYSRS_1545

| CodeBeamer reference: | APAVPCSR-3416133 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal TCU_Current_Veh_Posn from TCU

###### 18.1.1.33 VF_PF_SYSRS_1546

| CodeBeamer reference: | APAVPCSR-3416134 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal C-App_VPC_Actvn_Req from CGW.

###### 18.1.1.34 VF_PF_SYSRS_1547

| CodeBeamer reference: | APAVPCSR-3416135 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal VCU_Gear_State from VCU

###### 18.1.1.35 VF_PF_SYSRS_1548

| CodeBeamer reference: | APAVPCSR-3416136 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal ACM_Seat_Adj_Status from ACM

###### 18.1.1.36 VF_PF_SYSRS_1549

| CodeBeamer reference: | APAVPCSR-3416137 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal MHU_Park_Type_Req from MHU

###### 18.1.1.37 VF_PF_SYSRS_1550

| CodeBeamer reference: | APAVPCSR-3416138 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal C-App_Park_Type_Req from TCU

###### 18.1.1.38 VF_PF_SYSRS_1551

| CodeBeamer reference: | APAVPCSR-3416139 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal MHU_Nose-in_Req from MHU

###### 18.1.1.39 VF_PF_SYSRS_1552

| CodeBeamer reference: | APAVPCSR-3416140 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal MHU_Selected_Trajectory from MHU

###### 18.1.1.40 VF_PF_SYSRS_1553

| CodeBeamer reference: | APAVPCSR-3416141 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal DMS_DriverOccupancy from DMS

###### 18.1.1.41 VF_PF_SYSRS_1554

| CodeBeamer reference: | APAVPCSR-3416142 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal DMS_button_released from DMS 
Note: Set to True when the driver is inattentive

###### 18.1.1.42 VF_PF_SYSRS_1555

| CodeBeamer reference: | APAVPCSR-3416143 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal CMS_Occupancy from CMS

###### 18.1.1.43 VF_PF_SYSRS_1556

| CodeBeamer reference: | APAVPCSR-3416144 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal OTA_ APA_SW_Upd from OTA

###### 18.1.1.44 VF_PF_SYSRS_1557

| CodeBeamer reference: | APAVPCSR-3416145 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal Keyfob_VPC_Cancel_Req from Key Fob

###### 18.1.1.45 VF_PF_SYSRS_1558

| CodeBeamer reference: | APAVPCSR-3416146 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal C-App_VPC_Cancel_Req from TCU

###### 18.1.1.46 VF_PF_SYSRS_1559

| CodeBeamer reference: | APAVPCSR-3416147 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal IDB_Brake_Override_Level from IDB

###### 18.1.1.47 VF_PF_SYSRS_1560

| CodeBeamer reference: | APAVPCSR-3416148 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal Camera_Visibility_Status from Camera Module

###### 18.1.1.48 VF_PF_SYSRS_1561

| CodeBeamer reference: | APAVPCSR-3416149 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal BMS_Battery_SoC from BMS

###### 18.1.1.49 VF_PF_SYSRS_1562

| CodeBeamer reference: | APAVPCSR-3416150 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal ACC_Requested from ACC

###### 18.1.1.50 VF_PF_SYSRS_1563

| CodeBeamer reference: | APAVPCSR-3416151 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal BCM_Doors_Lock_Status from BCM

###### 18.1.1.51 VF_PF_SYSRS_1564

| CodeBeamer reference: | APAVPCSR-3416152 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal BCM_Trailer_Attachment_Status from BCM

###### 18.1.1.52 VF_PF_SYSRS_1565

| CodeBeamer reference: | APAVPCSR-3416153 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal Slope_exceeds_limitation from ESP

###### 18.1.1.53 VF_PF_SYSRS_1566

| CodeBeamer reference: | APAVPCSR-3416154 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall receive the signal MHU_Display_Slot_Ack from MHU

##### 18.1.2 Output Signals

| CodeBeamer reference: | APAVPCSR-3415783 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

###### 18.1.2.1 --

| CodeBeamer reference: | APAVPCSR-3416155 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |
| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal APA_Status in MHU.

###### 18.1.2.2 --

| CodeBeamer reference: | APAVPCSR-3416156 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |
| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal VPC_Parking_Active

###### 18.1.2.3 --

| CodeBeamer reference: | APAVPCSR-3416157 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |
| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal VPC_Status in MHU.

###### 18.1.2.4 --

| CodeBeamer reference: | APAVPCSR-3416158 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415559 | Pre-Maneuver Condition Check and Status Communication |
| --- | --- |
| APAVPC-3415560 | Pre-Maneuver Condition Failure Notification |

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal VPC_Maneuver_precond in MHU.

###### 18.1.2.5 --

| CodeBeamer reference: | APAVPCSR-3416159 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415545 | Remote Mode Selection: Occupancy detection |
| --- | --- |

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal VPC_MHU_Nose-in_feasible in MHU.

###### 18.1.2.6 --

| CodeBeamer reference: | APAVPCSR-3416160 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415545 | Remote Mode Selection: Occupancy detection |
| --- | --- |

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal VPC_MHU_Nose-out_feasible in MHU.

###### 18.1.2.7 --

| CodeBeamer reference: | APAVPCSR-3416161 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415554 | Selection of Control mode for V_Comfort Operation |
| --- | --- |

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal VPC_MHU_User_Comm_dev in MHU.

###### 18.1.2.8 --

| CodeBeamer reference: | APAVPCSR-3416162 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415545 | Remote Mode Selection: Occupancy detection |
| --- | --- |

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal VPC_Maneuver_precond in MHU.

###### 18.1.2.9 --

| CodeBeamer reference: | APAVPCSR-3416163 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415565 | Remote Mode Selection: Continuous Engagement via Mobile App |
| --- | --- |
| APAVPC-3415571 | Parking Feasibility Check |

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal VPC_MHU_Nose-in_Trajectory

###### 18.1.2.10 --

| CodeBeamer reference: | APAVPCSR-3416164 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415587 | Park-In Completion: Alignment Check |
| --- | --- |

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal VPC_SuccessTxt in MHU

###### 18.1.2.11 --

| CodeBeamer reference: | APAVPCSR-3416165 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415526 | Automated Alert Generation for VPARK System Performance Degradation |
| --- | --- |
| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal APA_Error_status in MHU.

###### 18.1.2.12 --

| CodeBeamer reference: | APAVPCSR-3416166 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (3)

| APAVPC-3415573 | System Recommendation |
| --- | --- |
| APAVPC-3415577 | Remote Mode Selection: Driver exit without Park brake |
| APAVPC-3415561 | Parking Slot Detection |

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal VPC_MHU_Nose-out_Trajectory

###### 18.1.2.13 --

| CodeBeamer reference: | APAVPCSR-3416167 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415580 | Trajectory Calculation Based on Driver’s Slot and Direction Selection |
| --- | --- |

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal Trace_Path in MHU.

###### 18.1.2.14 --

| CodeBeamer reference: | APAVPCSR-3416168 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (3)

| APAVPC-3415617 | Remote Interface Connectivity |
| --- | --- |
| APAVPC-3415559 | Pre-Maneuver Condition Check and Status Communication |
| APAVPC-3415601 | Remote Supervision During V_Comfort Park-Out via Mobile App |

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal VPC_C-App_Comm_Status in MHU.

###### 18.1.2.15 --

| CodeBeamer reference: | APAVPCSR-3416169 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415601 | Remote Supervision During V_Comfort Park-Out via Mobile App |
| --- | --- |
| APAVPC-3415617 | Remote Interface Connectivity |

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal VPC_Keyfob_Comm_Status in MHU.

###### 18.1.2.16 --

| CodeBeamer reference: | APAVPCSR-3416170 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal Reason_for_suspend in MHU.

###### 18.1.2.17 --

| CodeBeamer reference: | APAVPCSR-3416171 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415587 | Park-In Completion: Alignment Check |
| --- | --- |

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal VPC_Park-in_Finished_Status

###### 18.1.2.18 --

| CodeBeamer reference: | APAVPCSR-3416172 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415610 | Maneuver Completion Actions After Successful Park-Out |
| --- | --- |

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal VPC_Park-out_Finished_Status

###### 18.1.2.19 --

| CodeBeamer reference: | APAVPCSR-3416173 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal Reason_for_abort in MHU.

###### 18.1.2.20 --

| CodeBeamer reference: | APAVPCSR-3416174 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415580 | Trajectory Calculation Based on Driver’s Slot and Direction Selection |
| --- | --- |

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal VPC_Park_out_directions

###### 18.1.2.21 --

| CodeBeamer reference: | APAVPCSR-3416175 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415583 | Display Park-In Trajectory and progress |
| --- | --- |
| APAVPC-3415598 | Low-speed navigation: Initiation via MHU |

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal VPC_C-App_Park-out_Trajectory

###### 18.1.2.22 VF_PF_SYSRS_1566

| CodeBeamer reference: | APAVPCSR-3416176 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal VPC_MHU_Parking_Slot_Info in MHU

###### 18.1.2.23 VF_PF_SYSRS_1567

| CodeBeamer reference: | APAVPCSR-3416177 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal VPC_MHU_Nose-out_CompltnTime in MHU

###### 18.1.2.24 VF_PF_SYSRS_1568

| CodeBeamer reference: | APAVPCSR-3416178 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal VPC_MHU_Nose-in_CompltnTime in MHU

###### 18.1.2.25 VF_PF_SYSRS_1569

| CodeBeamer reference: | APAVPCSR-3416179 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal VPC_EPS_Steering_Ctrl_Req to EPS

###### 18.1.2.26 VF_PF_SYSRS_1570

| CodeBeamer reference: | APAVPCSR-3416180 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal VPC_IDB_Ctrl_Req

###### 18.1.2.27 VF_PF_SYSRS_1571

| CodeBeamer reference: | APAVPCSR-3416181 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal VPC_VCU_Gear_Ctrl_Req to VCU

###### 18.1.2.28 VF_PF_SYSRS_1572

| CodeBeamer reference: | APAVPCSR-3416182 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal VPC_VCU_Throttle_Ctrl_Req to VCU

###### 18.1.2.29 VF_PF_SYSRS_1573

| CodeBeamer reference: | APAVPCSR-3416183 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal VPC_IDB_Cmd

###### 18.1.2.30 VF_PF_SYSRS_1574

| CodeBeamer reference: | APAVPCSR-3416184 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal VPC_EPB_Cmd

###### 18.1.2.31 VF_PF_SYSRS_1575

| CodeBeamer reference: | APAVPCSR-3416185 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal VPC_EPS_Steering_Cmd

###### 18.1.2.32 VF_PF_SYSRS_1576

| CodeBeamer reference: | APAVPCSR-3416186 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal VPC_VCU_Throttle_Cmd to VCU

###### 18.1.2.33 VF_PF_SYSRS_1577

| CodeBeamer reference: | APAVPCSR-3416187 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal VPC_VCU_Gear_Cmd to VCU

###### 18.1.2.34 VF_PF_SYSRS_1578

| CodeBeamer reference: | APAVPCSR-3416188 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal VPC_EPS_Steering_Cmd

###### 18.1.2.35 VF_PF_SYSRS_1579

| CodeBeamer reference: | APAVPCSR-3416189 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-parking Comfort shall update the signal VPC_EPB_Ctrl_Req

#### 18.2 State Diagram:

| CodeBeamer reference: | APAVPCSR-3415719 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.2.1 --

| CodeBeamer reference: | APAVPCSR-3415784 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

#### 18.3 State Transitions:

| CodeBeamer reference: | APAVPCSR-3415720 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.3.1 --

| CodeBeamer reference: | APAVPCSR-3415785 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415542 | Path and Trajectory Planning: Traffic circulation |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

T1: APA Disabled To APA Enabled: 
When
MHU_APA_Req is Enable
AND 
OTA_ APA_SW_Upd is False
AND 
APA_Error_Status is False
Then
The APA system shall set APA_Status as APA Enabled.

##### 18.3.2 --

| CodeBeamer reference: | APAVPCSR-3415786 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415625 | System states in Drive mode |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

T2: APA Enabled To APA Disabled : 
When
MHU_APA_Req is Disable
OR 
OTA_APA_SW_Upd is True
AND 
APA_Error_Status is False
Then
The APA system shall set APA_Status as APA Disabled.

##### 18.3.3 --

| CodeBeamer reference: | APAVPCSR-3415787 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415626 | Vehicle mode to enable V_Comfort |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

Idle
When
    APA_Status is Enabled 
AND
    VCU_Ign_On is True
AND
    VCU_Veh_Spd is greater than SLOT_SCAN_SPEED
Then
   The APA system shall set APA_Status as Idle

##### 18.3.4 VF_PF_SYSRS_1580

| CodeBeamer reference: | APAVPCSR-3415788 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

Idle
When
APA_Status is Enabled 
AND
VCU_Ign_On is True
AND
VPC_Parking_Active is False
Then
The APA system shall set VPC_Status as Idle

##### 18.3.5 --

| CodeBeamer reference: | APAVPCSR-3415789 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| AVPCSR-3417038 | -- |
| --- | --- |
| APAVPC-3415541 | VPARK background scanning |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

T3:Idle To Slot-Searching:
When
APA_Status is Idle
AND 
VCU_Veh_Spd is less than or equal to SLOT_SCAN_SPEED
Then
The APA system shall set APA_Status as Slot Searching

##### 18.3.6 --

| CodeBeamer reference: | APAVPCSR-3415790 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415563 | Continuous Trajectory Recording |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

T4: Slot-Searching To Record Trajectory:
When
APA_Status is Slot-Searching
Then
APA system shall set APA_Status as Record Trajectory

##### 18.3.7 --

| CodeBeamer reference: | APAVPCSR-3415791 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415563 | Continuous Trajectory Recording |
| --- | --- |
| AVPCSR-3416560 | -- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

T5: Slot-Searching To Slot Display AND Auto-Maneuvering
When
APA_Status is Slot Searching
AND
VPC_Parking_Active is True
AND
VPC_Maneuver_Precond is True
AND
(MHU_Maneuver_Command is True AND DMS_DriverOccupancy is True)
OR
C-App_Maneuver_Command is True
AND
MHU_Park_Type_Req is Park-in
OR
C-App_Park_Type_Req is Park-in
Then
V-Parking Comfort shall set VPC_Status as Slot Display AND Auto-Maneuvering

##### 18.3.8 --

| CodeBeamer reference: | APAVPCSR-3415792 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

T6:Slot Display AND auto-maneuvering To Vehicle Standby
When
VPC_Status is Slot Display AND auto-maneuvering 
AND
Slot_Detected is True 
OR
Obstcl_Identified is True
OR
C-App_Maneuver_Command is False
OR
DMS_DriverOccupancy is False
OR
Distance reached is more than the MAX_DIST , Odometer_DistanceTrav is greater than the MAX_DIST 
Then
V-Parking Comfort shall set VPC_Status as Vehicle Standby

##### 18.3.9 --

| CodeBeamer reference: | APAVPCSR-3415793 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415564 | Low-speed forward navigation Execution |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

T7: Vehicle Standby to Slot Display AND auto-maneuvering
When
VPC_Status is Vehicle Standby 
AND
The user resumes auto-park by doing the activating command,C-App_Maneuver_Command is True within MAX_STANDBY_TIME
OR
Obstcl_Identified is False 
Then
V-Parking Comfort shall set VPC_Status as Slot Display AND auto-maneuvering

##### 18.3.10 --

| CodeBeamer reference: | APAVPCSR-3415794 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415577 | Remote Mode Selection: Driver exit without Park brake |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

T8: Vehicle Standby To Slot Confirmation
When
VPC_Status is Vehicle Standby
AND
VPC_MHU_Parking_Slot_Info sent to MHU 
AND 
MHU_Display_Slot_Ack is True from MHU 
AND
VPC_C-App_Parking_Slot_Info sent to C-App 
AND 
C-App_Display_Slot_Ack is True from C-App
Then 
V-Parking Comfort shall set VPC_Status as Slot Confirmation

##### 18.3.11 --

| CodeBeamer reference: | APAVPCSR-3415795 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415580 | Trajectory Calculation Based on Driver’s Slot and Direction Selection |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

T9: Slot Confirmation To PATH PLANNING and SELECTION
When
VPC_Status is Slot Confirmation
AND
C-App_Slot_Confrm is True
OR
MHU_Slot_Confrm is True
Then 
V-Parking Comfort shall set VPC_Status as PATH PLANNING and SELECTION

##### 18.3.12 --

| CodeBeamer reference: | APAVPCSR-3415796 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415581 | Persistent Obstacle: Abort maneuver by System |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

T10: Slot Confirmation To Slot Display AND auto-maneuvering
When
VPC_Status is Slot Confirmation
AND
C-App_Slot_Confrm is False
OR
MHU_Slot_Confrm is False
Then 
V-Parking Comfort shall set VPC_Status as Slot Display AND auto-maneuvering

##### 18.3.13 --

| CodeBeamer reference: | APAVPCSR-3415797 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415597 | Park-In Maneuver terminated by Remote Driver |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

T11: Reset To PATH PLANNING and SELECTION (Park-Out)
When
VPC _Status is Reset
AND
VPC_Parking_Active is True
AND 
MHU_Park_Type_Req is Park-out
OR
C-App_Park_Type_Req is Park-out
AND
Record_Avlbty is True, Recorded data is available
Then 
V-Parking Comfort shall set VPC_Status as PATH PLANNING and SELECTION

##### 18.3.14 --

| CodeBeamer reference: | APAVPCSR-3415798 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415601 | Remote Supervision During V_Comfort Park-Out via Mobile App |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

T12: PATH PLANNING and SELECTION To PATH MANOEUVRING/MOVING
When
VPC_Status is PATH PLANNING and SELECTION 
AND
User provides confirmation to start maneuvering in C-App, 
C-App_Maneuver_Command is True
OR 
MHU_Maneuver_Command is True
AND
VPC_Maneuver_Precond is true
Then
V-Parking Comfort shall set VPC_Status as Path Maneuvering/moving.
Note:
C-App Command need to be continously active for maneuvering

##### 18.3.15 --

| CodeBeamer reference: | APAVPCSR-3415799 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415602 | Deactivation: System Abort |
| --- | --- |
| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

T13: PATH MANOEUVRING/MOVING To Suspend
When 
VPC_Status is Path Maneuvering/moving
AND
Driver releases button in C-App, C-App_Maneuver_Command is False
OR
Obstcl_Identified is True 
Then
V-Parking Comfort shall set VPC_Status as Suspend

##### 18.3.16 --

| CodeBeamer reference: | APAVPCSR-3415800 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| AVPCSR-3417035 | -- |
| --- | --- |
| APAVPC-3415602 | Deactivation: System Abort |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

T14: Suspend To PATH MANOEUVRING/MOVING 
When 
VPC_Status is Suspend
AND
The user resumes auto-park by doing the activating command , C-App_Maneuver_Command is True within MAX_SUSPEND_TIME
OR
Obstcl_Identified is False within MAX_SUSPEND_TIME 
Then
V-Parking Comfort shall set VPC_Status as Path Maneuvering/moving

##### 18.3.17 --

| CodeBeamer reference: | APAVPCSR-3415801 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415585 | Status Updates during Park-in maneuver: Remote |
| --- | --- |
| APAVPC-3415584 | Park-In Maneuver Initiation: Control mode change |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

T15: Path Maneuvering/moving To Finished
When 
VPC_Status is Path Maneuvering/moving
AND
Target_Coordinate equals to TCU_Current_Veh_Posn (The planned path is traced successfully)
Then
V-Parking Comfort shall set VPC_Status as Finished

##### 18.3.18 --

| CodeBeamer reference: | APAVPCSR-3415802 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415585 | Status Updates during Park-in maneuver: Remote |
| --- | --- |
| APAVPC-3415589 | Park-In Completion Activities: Remote driver |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

T16: Finished To Completed
When 
VPC_Status is Finished 
AND
VPC_Park in_Finished_Status is success
OR
VPC_Park out_Finished_Status is success
Then
V-Parking Comfort shall set VPC_Status as Completed

##### 18.3.19 --

| CodeBeamer reference: | APAVPCSR-3415803 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

No Upstream References

No Downstream References

No Associations

Description

T17: Completed To Idle
When 
VPC_Status is Completed for more than MAX_TIME_CMPLTD
Then
V-Parking Comfort shall set VPC_Status as Idle

##### 18.3.20 --

| CodeBeamer reference: | APAVPCSR-3415804 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |
| AVPCSR-3417124 | -- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

T18: Path Tracing Maneuver To Abort
When 
VPC_Status is Path Maneuvering/moving or Suspend or Finished 
AND
VPC_Fault_Status is True
OR 
Obstcl_Identified is True for more than MAX_SUSPEND_TIME
OR
VPC_Park in_Finished_Status is Failed
OR
VPC_Park out_Finished_Status is Failed
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.3.21 --

| CodeBeamer reference: | APAVPCSR-3415805 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

No Upstream References

No Downstream References

No Associations

Description

T19: Abort To Idle
When 
VPC_Status is Abort 
AND
The driver takes control over the vehicle, 
IDB_Ctrl_Status is Driver Control
AND 
EPB_Ctrl_Status is Driver Control
AND
EPS_Steering_Ctrl_Status is Driver Control
AND
VCU_Throttle_Ctrl_Status is Driver Control
AND
VCU_Gear_Ctrl_Status is Driver Control
Then
V-Parking Comfort shall set VPC_Status as Idle

##### 18.3.22 --

| CodeBeamer reference: | APAVPCSR-3415806 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| AVPCSR-3417010 | -- |
| --- | --- |
| AVPCSR-3417011 | -- |

No Downstream References

No Associations

Description

T20: APA Enabled To System Fault/Error
When 
APA_Status is APA Enabled
AND
APA_Error_Status is True
Then
APA shall set APA_Status as System Fault/Error

##### 18.3.23 --

| CodeBeamer reference: | APAVPCSR-3415807 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415625 | System states in Drive mode |
| --- | --- |

No Downstream References

No Associations

Description

T21:System Fault/Error To APA Enabled
When 
APA_Status is System Fault/Error
AND
APA_Error_Status is resolved,APA_Error_Status is False
AND
OTA_APA_SW_Upd is False
Then
APA shall set APA_Status as APA Enabled

##### 18.3.24 --

| CodeBeamer reference: | APAVPCSR-3415808 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

T22: Slot Selection Maneuver To ABORT
When 
VPC_Status is Slot Selection Maneuver
AND
VPC_Fault_Status is True during Maneuver
OR 
Obstcl_Identified is True for more than MAX_SUSPEND_TIME
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.3.25 --

| CodeBeamer reference: | APAVPCSR-3415809 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415541 | VPARK background scanning |
| --- | --- |

No Downstream References

No Associations

Description

T23: Slot Searching To Idle
When
APA_Status is Slot Searching
AND
VCU_Veh_Spd is greater than SLOT_SCAN_SPEED
Then
V-Parking Comfort shall set APA_Status is Idle

##### 18.3.26 --

| CodeBeamer reference: | APAVPCSR-3415810 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415581 | Persistent Obstacle: Abort maneuver by System |
| --- | --- |
| APAVPC-3415580 | Trajectory Calculation Based on Driver’s Slot and Direction Selection |

Downstream References (1)

| APASAD-3412216 | Sequence Diagram |
| --- | --- |

No Associations

Description

T24:Path-planning and selection To Slot Display and Auto-Maneuvering
When
MHU_Park_Type_Req is Park-in
OR
C-App_Park_Type_Req is Park-in
AND
VPC_Status is Path-planning and selection
AND
Nose-in_feasible is False 
AND 
Nose-out_feasible is False 
Then
V-Parking Comfort shall send VPC_Parking_NotFeasibleTxt to MHU


Note: Inform driver that no feasible path is available to reach the selected slot and prompt to select another slot.

##### 18.3.27 VF_PF_SYSRS_1581

| CodeBeamer reference: | APAVPCSR-3415811 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

T24:Path-planning and selection To Slot Display and Auto-Maneuvering
When
Park_type is Park in
AND
VPC_Status is Path-planning and selection
AND
Nose-in_feasible is False 
AND
Nose-out_feasible is False
Then
V-Parking Comfort shall set VPC_Status as Slot Display and Auto-Maneuvering

#### 18.4 APA Enable/Disable:

| CodeBeamer reference: | APAVPCSR-3415721 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.4.1 Description

| CodeBeamer reference: | APAVPCSR-3415812 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

The Automatic Parking Assist (APA) system supports vehicle maneuvering during parking operations. Drivers have the option to enable or disable APA. To utilize the Comfort feature, APA must be turned on.

##### 18.4.2 --

| CodeBeamer reference: | APAVPCSR-3415813 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415501 | Actuator Command Transmission and System Responsiveness |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

APA Functionality shall be Enabled/Disabled by user through any of the following:(refer T1)
MHU 
Dedicated Hard button(if available)

#### 18.5 Slot- Searching:

| CodeBeamer reference: | APAVPCSR-3415722 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.5.1 Descrption

| CodeBeamer reference: | APAVPCSR-3415814 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

Slot-Searching detects and classifies available parking spaces within a parking facility, making it easier for users to find open spots. The Comfort feature relies on Slot-Searching being active to function properly.

##### 18.5.2 Input Signals

| CodeBeamer reference: | APAVPCSR-3415815 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

VCU_Veh_Spd
Camer_Input
Sensor_Input

##### 18.5.3 --

| CodeBeamer reference: | APAVPCSR-3415816 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415561 | Parking Slot Detection |
| --- | --- |

Downstream References (1)

| APASAD-3412216 | Sequence Diagram |
| --- | --- |

No Associations

Description

V-Parking comfort shall identify and categorize the Parking slots with and without markings.

##### 18.5.4 --

| CodeBeamer reference: | APAVPCSR-3415817 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (4)

| APAVPC-3415492 | Slot Detection Capability |
| --- | --- |
| APAVPC-3415571 | Parking Feasibility Check |
| APAVPC-3415565 | Remote Mode Selection: Continuous Engagement via Mobile App |
| APAVPC-3415561 | Parking Slot Detection |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When 
The APA system is enabled 
AND
VCU_Veh_Spd is less than SLOT_SCAN_SPEED
Then
The APA System shall receive input from Camera(Camera_Input) and Sensor (Sensor_Input) to scan the surroundings automatically in the background

#### 18.6 V-Parking Comfort Active:

| CodeBeamer reference: | APAVPCSR-3415723 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.6.1 Descrption

| CodeBeamer reference: | APAVPCSR-3415818 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

Indicates that the vehicle's remote parking system is active, guiding the user through Auto or Manual slot selection based on pre-condition status and user input.

##### 18.6.2 Input Signals

| CodeBeamer reference: | APAVPCSR-3415819 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

MHU_VPC_Actvn_Req
C-App_VPC_Actvn_Req
BCM_STAT_DoorAjarFL
BCM_STAT_DoorAjarFR
BCM_STAT_DoorAjarRL
BCM_STAT_DoorAjarRR
BCM_STAT_BonnetAjar
BCM_STAT_Mirror
BCM_STAT_TrunkAjar
Sensor_Input_Status
Camera_Input_Status
MHU_Parking_Mode
C-App_Parking_Mode

##### 18.6.3 Output Signals

| CodeBeamer reference: | APAVPCSR-3415820 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

VPC_Maneuver_precond
VPC_Parking_Active
VPC_Precond 
VPC_StartInstrText
VPC_Parking_Mode_Req

##### 18.6.4 Internal Signals

| CodeBeamer reference: | APAVPCSR-3415821 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.6.5 VF_PF_SYSRS_1582

| CodeBeamer reference: | APAVPCSR-3415822 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

Upstream References (2)

| APAVPC-3415543 | V_Comfort Park-In Selection via MHU |
| --- | --- |
| APAVPC-3415544 | Communicate activation status to driver |

No Downstream References

No Associations

Description

When
BCM_STAT_DoorAjarFL is Closed (Door Front Left Status is closed)
AND
BCM_STAT_DoorAjarFR is Closed (Door Front Right Status is closed)
AND
BCM_STAT_DoorAjarRL is Closed (Door Rear Left Status is closed)
AND
BCM_STAT_DoorAjarRR is Closed (Door Rear Right Status is closed)
AND
BCM_STAT_BonnetAjar is Closed (Bonnet Status)
AND
BCM_STAT_Mirror is Unfold (Mirrors are unfolded)
AND 
BCM_STAT_TrunkAjar is Closed (Trunk Ajar Door Status)
AND
Sensor_Input_Status is valid
AND
Camera_Input_Status is valid
Then
V-Parking System shall set VPC_Precond as True

##### 18.6.6 --

| CodeBeamer reference: | APAVPCSR-3415823 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415543 | V_Comfort Park-In Selection via MHU |
| --- | --- |
| APAVPC-3415544 | Communicate activation status to driver |

Downstream References (1)

| APASAD-3412216 | Sequence Diagram |
| --- | --- |

No Associations

Description

When 
The user presses the V-Parking Comfort active button in MHU Interface
MHU_VPC_Actvn_Req is True
AND
C-App_VPC_Actvn_Req is True
AND
VPC_Precond is True
Then
The APA system shall set VPC_Parking_Active as true.

##### 18.6.7 --

| CodeBeamer reference: | APAVPCSR-3415824 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (3)

| APAVPC-3415543 | V_Comfort Park-In Selection via MHU |
| --- | --- |
| APAVPC-3415544 | Communicate activation status to driver |
| APAVPC-3415550 | Park In Mission: Comply with Legal Driving Side |

Downstream References (1)

| APASAD-3412216 | Sequence Diagram |
| --- | --- |

No Associations

Description

When 
The user presses the V-Parking Comfort active button in MHU Interface,
MHU_VPC_Actvn_Req is True
OR
C-App_VPC_Actvn_Req is True
AND
VPC_Precond is False
Then
The APA system shall set VPC_Parking_Active as false

##### 18.6.8 --

| CodeBeamer reference: | APAVPCSR-3415825 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415558 | Post selection of V_Comfort: Align vehicle using Perception Info |
| --- | --- |
| APAVPC-3415557 | Remote Mode Selection: Request Driver exit |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When 
VPC_Parking_Active is True
Then
The V-Parking Comfort shall send VPC_StartInstrText to instruct the driver to perform the following 
actions like:
- Align the vehicle with the driveway
- Engage Parking brake
- Release brake and throttle controls
- Handoff the Steering
- Lock the doors
- Take Remote device and other belongings (remote activation)
- Roll up the windows

##### 18.6.9 --

| CodeBeamer reference: | APAVPCSR-3415826 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415551 | Reuse |
| --- | --- |
| APAVPC-3415576 | Remote Mode Selection: Request User to open C-App |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When 
VPC_Parking_Active is True
Then
V-Parking Comfort shall send VPC_Parking_Mode_Req to choose between AUTO or MANUAL
Note:
When the Park in is activated, the VPC shall provide the user to select the remote Park in options 
AUTO
MANUAL

##### 18.6.10 --

| CodeBeamer reference: | APAVPCSR-3415827 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415551 | Reuse |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When 
VPC_Parking_Active is True
AND
MHU_Parking_Mode is AUTO
OR 
C-App_Parking_Mode is AUTO
Then 
V-Parking Comfort shall set VPC_Recommend_Slot as first available slot(Slot_ID )
Note:
When the remote Park in option is AUTO, the VPC shall automatically recommend the first available slot

##### 18.6.11 --

| CodeBeamer reference: | APAVPCSR-3415828 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415552 | Path and Trajectory Planning: Drivable Area Topology |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When 
VPC_Parking_Active is True
Then
V-Parking Comfort shall set VPC_Parking_Mode as AUTO

Note:
When the Park in is activated, the VPC shall provide AUTO Selection as the default option.

##### 18.6.12 --

| CodeBeamer reference: | APAVPCSR-3415829 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415551 | Reuse |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When 
VPC_Parking_Active is True
AND
MHU_Parking_Mode is MANUAL
OR 
C-App_Parking_Mode is MANUAL
Then
V-Parking Comfort shall send VPC_SlotSelnTxt to prompt the driver to select a slot 

Note:
When the remote Park in option is MANUAL, the VPC system shall ask the driver to select the slot and the desired direction

#### 18.7 Park in/Park- out Selection:

| CodeBeamer reference: | APAVPCSR-3415724 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.7.1 Descrption

| CodeBeamer reference: | APAVPCSR-3415830 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

Lets the system choose Park-in or Park-out mode, setting Park-in automatically when VPC is active and all conditions are met.

##### 18.7.2 Input Signals

| CodeBeamer reference: | APAVPCSR-3415831 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

MHU_Park_Type_Req
C-App_Park_Type_Req

##### 18.7.3 Output Signals

| CodeBeamer reference: | APAVPCSR-3415832 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

VPC_Parking_Active

##### 18.7.4 Internal Signals

| CodeBeamer reference: | APAVPCSR-3415833 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

Park_type

##### 18.7.5 --

| CodeBeamer reference: | APAVPCSR-3415834 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415551 | Reuse |
| --- | --- |

Downstream References (1)

| APASAD-3412216 | Sequence Diagram |
| --- | --- |

No Associations

Description

When
VPC_Parking_Active is true
AND
MHU_Park_Type_Req is Park-in
OR
C-App_Park_Type_Req is Park-in
Then
V-Parking Comfort shall set Park_type as Park-in

#### 18.8 Park in:

| CodeBeamer reference: | APAVPCSR-3415725 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.8.1 Slot Selection Maneuver:

| CodeBeamer reference: | APAVPCSR-3415835 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.8.2 Start Recording:

| CodeBeamer reference: | APAVPCSR-3415836 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

###### 18.8.2.1 Descrption

| CodeBeamer reference: | APAVPCSR-3416190 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

Begins recording the vehicle’s movement path during auto-maneuver when VPC is active and conditions are met.

###### 18.8.2.2 Output Signals

| CodeBeamer reference: | APAVPCSR-3416191 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

VPC_Status
APA_Status

###### 18.8.2.3 --

| CodeBeamer reference: | APAVPCSR-3416192 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415564 | Low-speed forward navigation Execution |
| --- | --- |

Downstream References (1)

| APASAD-3412216 | Sequence Diagram |
| --- | --- |

No Associations

Description

When
APA_Status is Record Trajectory
Then
The V-Parking Comfort shall start recording the trajectory traced during the auto-maneuver.

##### 18.8.3 Slot Display and Auto-Maneuvering:

| CodeBeamer reference: | APAVPCSR-3415837 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

###### 18.8.3.1 Descrption

| CodeBeamer reference: | APAVPCSR-3416193 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

Displays categorized parking slots and initiates automated maneuvering with safe trajectory and speed, while sharing slot data with the companion app when VPC is active and conditions are met.

###### 18.8.3.2 Input Signals

| CodeBeamer reference: | APAVPCSR-3416194 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

MHU_Park_Type_Req
C-App_Park_Type_Req
IDB_Ctrl_Status
EPB_Ctrl_Status
EPS_Steering_Ctrl_Status
VCU_Throttle_Ctrl_Status
VCU_Gear_Ctrl_Status
MHU_Parking_Mode
C-App_Parking_Mode
MHU_Maneuver_Command
DMS_DriverOccupancy
C-App_Maneuver_Command

###### 18.8.3.3 Output Signals

| CodeBeamer reference: | APAVPCSR-3416195 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

VPC_Status
VPC_Maneuver_Precond
VPC_EPB_Ctrl_Req
VPC_EPS_Steering_Ctrl_Req
VPC_IDB_Ctrl_Req
VPC_VCU_Gear_Ctrl_Req
VPC_VCU_Throttle_Ctrl_Req
VPC_IDB_Cmd
VPC_EPB_Cmd
VPC_EPS_Steering_Cmd
VPC_VCU_Throttle_Cmd
VPC_VCU_Gear_Cmd 
VPC_Parking_Slot_Info
VPC_Recommend_Slot

###### 18.8.3.4 Internal Signals

| CodeBeamer reference: | APAVPCSR-3416196 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

Slot_Detected
Obstcl_Identified

###### 18.8.3.5 VF_PF_SYSRS_1583

| CodeBeamer reference: | APAVPCSR-3416197 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

When 
VPC_Status is Slot Display and Auto-Maneuvering
AND
VPC_Maneuver_Precond is True
AND
MHU_Park_Type_Req is Park-in
OR
C-App_Park_Type_Req is Park-in
Then
The V-Parking Comfort shall send VPC_EPB_Ctrl_Req as True to request control of the park brake

###### 18.8.3.6 VF_PF_SYSRS_1584

| CodeBeamer reference: | APAVPCSR-3416198 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

When 
VPC_Status is Slot Display and Auto-Maneuvering
AND
VPC_Maneuver_Precond is True
AND
MHU_Park_Type_Req is Park-in
OR
C-App_Park_Type_Req is Park-in
Then
The V-Parking Comfort shall send VPC_EPS_Steering_Ctrl_Req as True to request control of the Steering

###### 18.8.3.7 VF_PF_SYSRS_1585

| CodeBeamer reference: | APAVPCSR-3416199 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

When 
VPC_Status is Slot Display and Auto-Maneuvering
AND
VPC_Maneuver_Precond is True
AND
MHU_Park_Type_Req is Park-in
OR
C-App_Park_Type_Req is Park-in
Then
The V-Parking Comfort shall send VPC_IDB_Ctrl_Req (True) to request control of the brake

###### 18.8.3.8 VF_PF_SYSRS_1586

| CodeBeamer reference: | APAVPCSR-3416200 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

When 
VPC_Status is Slot Display and Auto-Maneuvering
AND
VPC_Maneuver_Precond is True
AND
MHU_Park_Type_Req is Park-in
OR
C-App_Park_Type_Req is Park-in
Then
The V-Parking Comfort shall send VPC_VCU_Gear_Ctrl_Req (True) to request control of the gear selector

###### 18.8.3.9 VF_PF_SYSRS_1587

| CodeBeamer reference: | APAVPCSR-3416201 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

When 
VPC_Status is Slot Display and Auto-Maneuvering
AND
VPC_Maneuver_Precond is True
AND
MHU_Park_Type_Req is Park-in
OR
C-App_Park_Type_Req is Park-in
Then
The V-Parking Comfort shall send VPC_VCU_Throttle_Ctrl_Req (True) to request control of the throttle

###### 18.8.3.10 VF_PF_SYSRS_1588

| CodeBeamer reference: | APAVPCSR-3416202 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

When 
VPC_Status is Slot Display and Auto-Maneuvering
AND
VPC_Maneuver_Precond is True
AND
MHU_Park_Type_Req is Park-in
OR
C-App_Park_Type_Req is Park-in
AND
IDB_Ctrl_Status is System Control
AND 
EPB_Ctrl_Status is System Control
AND
EPS_Steering_Ctrl_Status is System Control
AND
VCU_Throttle_Ctrl_Status is System Control
AND
VCU_Gear_Ctrl_Status is System Control
Then
The V-Parking Comfort shall send VPC_IDB_Cmd to control brake

###### 18.8.3.11 VF_PF_SYSRS_1589

| CodeBeamer reference: | APAVPCSR-3416203 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

When 
VPC_Status is Slot Display and Auto-Maneuvering
AND
VPC_Maneuver_Precond is True
AND
IDB_Ctrl_Status is System Control
AND 
EPB_Ctrl_Status is System Control
AND
EPS_Steering_Ctrl_Status is System Control
AND
VCU_Throttle_Ctrl_Status is System Control
AND
VCU_Gear_Ctrl_Status is System Control
Then
The V-Parking Comfort shall send VPC_EPB_Cmd to control Parking brake

###### 18.8.3.12 VF_PF_SYSRS_1590

| CodeBeamer reference: | APAVPCSR-3416204 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

When 
VPC_Status is Slot Display and Auto-Maneuvering
AND
VPC_Maneuver_Precond is True
AND
MHU_Park_Type_Req is Park-in
OR
C-App_Park_Type_Req is Park-in
AND
IDB_Ctrl_Status is System Control
AND 
EPB_Ctrl_Status is System Control
AND
EPS_Steering_Ctrl_Status is System Control
AND
VCU_Throttle_Ctrl_Status is System Control
AND
VCU_Gear_Ctrl_Status is System Control
Then
The V-Parking Comfort shall send VPC_EPS_Steering_Cmd to EPS to Control Steering

###### 18.8.3.13 VF_PF_SYSRS_1591

| CodeBeamer reference: | APAVPCSR-3416205 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

When 
VPC_Status is Slot Display and Auto-Maneuvering
AND
VPC_Maneuver_Precond is True
AND
MHU_Park_Type_Req is Park-in
OR
C-App_Park_Type_Req is Park-in
AND
IDB_Ctrl_Status is System Control
AND 
EPB_Ctrl_Status is System Control
AND
EPS_Steering_Ctrl_Status is System Control
AND
VCU_Throttle_Ctrl_Status is System Control
AND
VCU_Gear_Ctrl_Status is System Control
Then
The V-Parking Comfort shall send VPC_VCU_Throttle_Cmd to VCU to control throttle

###### 18.8.3.14 VF_PF_SYSRS_1592

| CodeBeamer reference: | APAVPCSR-3416206 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

When 
VPC_Status is Slot Display and Auto-Maneuvering
AND
VPC_Maneuver_Precond is True
AND
MHU_Park_Type_Req is Park-in
OR
C-App_Park_Type_Req is Park-in
AND
IDB_Ctrl_Status is System Control
AND 
EPB_Ctrl_Status is System Control
AND
EPS_Steering_Ctrl_Status is System Control
AND
VCU_Throttle_Ctrl_Status is System Control
AND
VCU_Gear_Ctrl_Status is System Control
Then
The V-Parking Comfort shall send VPC_VCU_Gear_Cmd to VCU to Control gear selector

###### 18.8.3.15 --

| CodeBeamer reference: | APAVPCSR-3416207 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

In case of predefined slots with markings in parking area, the identified free slot is highlighted in C-APP and the driver shall confirm the slot.

###### 18.8.3.16 --

| CodeBeamer reference: | APAVPCSR-3416208 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415577 | Remote Mode Selection: Driver exit without Park brake |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When
VPC_Status is Slot Display and Auto-Maneuvering
AND
Slot_Detected is True 
AND
No Obstacles Identified in the free slot , Obstcl_Identified is False 
AND
VCU_Throttle_Ctrl_Status is System Control
AND
Slot width is greater than vehicle_width + X_OFFSET
AND
Slot length is greater than Vehicle_length +Y_OFFSET
Then
The V-Parking Comfrot shall send VPC_Parking_Slot_Info with slot infromations to display in MHU and C-App

Note: Slot information include Slot type(Perpendicular, Parallel, Angular), Slot length and width, Slot Position (Position with respect to the vehicle), Slot validity(not reserved, disabled, pregnant, senior citizen, EV charging point (if identified from sensor data)

###### 18.8.3.17 VF_PF_SYSRS_1593

| CodeBeamer reference: | APAVPCSR-3416209 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

When
VPC_Status is Slot Display and Auto-Maneuvering
AND
Slot_Detected is True 
AND
No Obstacles Identified in the free slot , Obstcl_Identified is False 
AND
Slot width is greater than VEH_WIDTH+ X_OFFSET
AND
Slot length is greater than VEH_LEN + Y_OFFSET
AND
MHU_Parking_Mode is AUTO
OR 
C-App_Parking_Mode is AUTO
Then
V-Parking Comfort shall recommend a slot (VPC_Recommend_Slot)to park-in

###### 18.8.3.18 --

| CodeBeamer reference: | APAVPCSR-3416210 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall scan and recognize reserved parking slot and highlight in C-APP for the user to select if it is applicable.

###### 18.8.3.19 --

| CodeBeamer reference: | APAVPCSR-3416211 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415492 | Slot Detection Capability |
| --- | --- |

Downstream References (1)

| APASAD-3412216 | Sequence Diagram |
| --- | --- |

No Associations

Description

The APA system shall perform slot detection using the feature in the following scenarios with a minimum target KPI of 92%, ensuring improved detection reliability.
1. Parallel (USS)_Vehicle Length+1m 
2. Perpendicular(USS)_Vehicle Width+0.8m
3. Fishbone (USS)_Vehicle Width+0.8m 
4. Parallel (Line)
5. Perpendicular(Line)
6. Fishbone(Line)

###### 18.8.3.20 --

| CodeBeamer reference: | APAVPCSR-3416212 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415577 | Remote Mode Selection: Driver exit without Park brake |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When
VPC_Status is Slot Display and Auto-Maneuvering
AND
VPC_Maneuver_Precond is True
AND
Obstcl_Identified is False
AND
(MHU_Maneuver_Command is True AND DMS_DriverOccupancy is True)
OR 
C-App_Maneuver_Command is True 
Then
The V-Parking Comfort shall send VPC_VCU_Throttle_Cmd to maneuver along the Trace_Path coordinates at a speed of less than or equal to MAX_AUTO_MANEUVERING_SPEED [10kmph +or- 2kmph(tolerance)]
Note: 
C-App Command need to be continously active for maneuvering

##### 18.8.4 Vehicle Standby:

| CodeBeamer reference: | APAVPCSR-3415838 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

###### 18.8.4.1 Descrption

| CodeBeamer reference: | APAVPCSR-3416213 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

Brings the vehicle to a complete stop (0 km/h) when in standby mode, ensuring safety while VPC is active and conditions are met.

###### 18.8.4.2 Input Signals

| CodeBeamer reference: | APAVPCSR-3416214 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

###### 18.8.4.3 Output Signals

| CodeBeamer reference: | APAVPCSR-3416215 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

VPC_Status
VPC_IDB_Cmd
VPC_VCU_Gear_State
VPC_EPB_Cmd

###### 18.8.4.4 Internal Signals

| CodeBeamer reference: | APAVPCSR-3416216 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

###### 18.8.4.5 --

| CodeBeamer reference: | APAVPCSR-3416217 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (1)

| APASAD-3412216 | Sequence Diagram |
| --- | --- |

No Associations

Description

When
VPC_Status is Vehicle Standby
Then
V-Parking Comfort shall send VPC_IDB_Cmd (Applied) to IDB --> To bring the vehicle to a stop

###### 18.8.4.6 VF_PF_SYSRS_1594

| CodeBeamer reference: | APAVPCSR-3416218 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

When
VPC_Status is Vehicle Standby
Then
V-Parking Comfort shall send VPC_VCU_Gear_State(Park) to bring the vehicle to stationary

###### 18.8.4.7 VF_PF_SYSRS_1595

| CodeBeamer reference: | APAVPCSR-3416219 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

When
VPC_Status is Vehicle Standby
Then
V-Parking Comfort shall send VPC_EPB_Cmd (Applied) to EPB --> To bring the vehicle to a stop

##### 18.8.5 Slot Confirmation

| CodeBeamer reference: | APAVPCSR-3415839 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

###### 18.8.5.1 Descrption

| CodeBeamer reference: | APAVPCSR-3416220 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

Allows the user to confirm or select a parking slot through the companion app (C-APP), enabling the system to automatically park the vehicle in the chosen slot when VPC is active and all conditions are met.

###### 18.8.5.2 Input Signals

| CodeBeamer reference: | APAVPCSR-3416221 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

MHU_Parking_Mode
C-App_Parking_Mode
C-App_Slot_Confrm
MHU_Slot_Confrm

###### 18.8.5.3 Output Signals

| CodeBeamer reference: | APAVPCSR-3416222 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

VPC_Status
VPC_Recommend_Slot
VPC_Slot_Confrm_Req

###### 18.8.5.4 Internal Signals

| CodeBeamer reference: | APAVPCSR-3416223 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

###### 18.8.5.5 --

| CodeBeamer reference: | APAVPCSR-3416224 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

The VPC shall provide AI-generated parking spot recommendations based on multi-criteria optimization (space constraints, vehicle dimensions, and optimal positioning for a smooth exit), while allowing drivers to manually select alternate locations via Mobile application using intuitive drag-and-drop functionality with real-time environmental data.

###### 18.8.5.6 --

| CodeBeamer reference: | APAVPCSR-3416225 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415584 | Park-In Maneuver Initiation: Control mode change |
| --- | --- |
| APAVPC-3415500 | Parking Parameters Input from Mobile Interface |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When
VPC_Status is Slot Confirmation
AND
MHU_Parking_Mode is MANUAL
OR 
C-App_Parking_Mode is MANUAL
AND
C-App_Slot_Confrm is True
OR
MHU_Slot_Confrm is True
Then
The V-Parking Comfort shall set VPC_Status as PATH PLANNING and SELECTION

###### 18.8.5.7 VF_PF_SYSRS_1596

| CodeBeamer reference: | APAVPCSR-3416226 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

When
VPC_Status is Slot Confirmation
AND
MHU_Parking_Mode is AUTO
OR 
C-App_Parking_Mode is AUTO
AND
VPC_Recommend_Slot is first available slot(Slot_ID) 
Then
V-Parking Comfort shall send VPC_Slot_Confrm_Req as True to request user confirmation for the recommended slot

#### 18.9 Park out:

| CodeBeamer reference: | APAVPCSR-3415726 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.9.1 User preferred communication device:

| CodeBeamer reference: | APAVPCSR-3415840 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

###### 18.9.1.1 Descrption

| CodeBeamer reference: | APAVPCSR-3416227 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

Park-out can be initiated via C-App with user confirmation. The system communicates with the selected device and proceeds with auto-maneuvering only when all conditions are met.

###### 18.9.1.2 Input Signals

| CodeBeamer reference: | APAVPCSR-3416228 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

C-App_VPC_Actvn_Req 
Keyfob_Maneuver_Command 
C-App_Maneuver_Command
MHU_Maneuver_Command
DMS_DriverOccupancy

###### 18.9.1.3 Output Signals

| CodeBeamer reference: | APAVPCSR-3416229 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

APA_Status
VPC_Status
VPC_User_Comm_dev
VPC_MHU_User_Comm_dev 
VPC_C-App_Communication_Status
VPC_Keyfob_Communication_Status

###### 18.9.1.4 --

| CodeBeamer reference: | APAVPCSR-3416230 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415554 | Selection of Control mode for V_Comfort Operation |
| --- | --- |
| APAVPC-3415617 | Remote Interface Connectivity |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When
VPC_Parking_Active is True
AND
C-App_VPC_Actvn_Req is True
Then
V-Parking Comfort shall ensure communication with C-App.

###### 18.9.1.5 --

| CodeBeamer reference: | APAVPCSR-3416231 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415617 | Remote Interface Connectivity |
| --- | --- |
| APAVPC-3415554 | Selection of Control mode for V_Comfort Operation |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

When
VPC_Status is active
AND
Keyfob_VPC_Actvn_Req
Then
V-Parking Comfort shall ensure communication with key-fob.

###### 18.9.1.6 --

| CodeBeamer reference: | APAVPCSR-3416232 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415554 | Selection of Control mode for V_Comfort Operation |
| --- | --- |

No Downstream References

No Associations

Description

When
APA_Status is APA Enabled 
AND
C-App_VPC_Actvn_Req is True
Then
V-parking comfort shall set User_Comm_dev as C-App

###### 18.9.1.7 --

| CodeBeamer reference: | APAVPCSR-3416233 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415554 | Selection of Control mode for V_Comfort Operation |
| --- | --- |

Downstream References (1)

| APASAD-3412216 | Sequence Diagram |
| --- | --- |

No Associations

Description

When
APA_Status is APA Enabled 
AND
Keyfob_Maneuver_Command is True
Then
V-parking comfort shall set User_Comm_dev as key fob

###### 18.9.1.8 --

| CodeBeamer reference: | APAVPCSR-3416234 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415594 | Driver Proximity Requirement for Remote Control |
| --- | --- |
| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |

Downstream References (1)

| APASAD-3412216 | Sequence Diagram |
| --- | --- |

No Associations

Description

When
The user mobile is not identified within the MAX_CONNECT_RANGE mts
Then
V-Parking comfort shall set VPC_C-App_Communication_Status as failed

###### 18.9.1.9 --

| CodeBeamer reference: | APAVPCSR-3416235 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (3)

| APAVPC-3415554 | Selection of Control mode for V_Comfort Operation |
| --- | --- |
| APAVPC-3415545 | Remote Mode Selection: Occupancy detection |
| APAVPC-3415594 | Driver Proximity Requirement for Remote Control |

Downstream References (1)

| APASAD-3412216 | Sequence Diagram |
| --- | --- |

No Associations

Description

When
The user mobile is identified within the MAX_CONNECT_RANGE mts and connected
Then
V-Parking Comfort shall set VPC_C-App_Communication_Status as Success

###### 18.9.1.10 --

| CodeBeamer reference: | APAVPCSR-3416236 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415594 | Driver Proximity Requirement for Remote Control |
| --- | --- |

No Downstream References

No Associations

Description

When
The key fob is not identified within the MAX_CONNECT_RANGE mts
Then
V-Parking comfort shall set VPC_Keyfob_Communication_Status as failed

###### 18.9.1.11 --

| CodeBeamer reference: | APAVPCSR-3416237 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415584 | Park-In Maneuver Initiation: Control mode change |
| --- | --- |

No Downstream References

No Associations

Description

When
The key fob is identified within the MAX_CONNECT_RANGE mts and connected
Then
V-Parking comfort shall set VPC_Keyfob_Communication_Status as Success

###### 18.9.1.12 --

| CodeBeamer reference: | APAVPCSR-3416238 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415585 | Status Updates during Park-in maneuver: Remote |
| --- | --- |
| APAVPC-3415607 | Real-Time Trajectory and Vehicle Status Display - Remote Park-Out |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When
VPC_Status is Path-planning and Selection
AND
VPC_C-App_Communication_Status is Success
Then
V-Parking Comfort shall send VPC_C-App_Communication_Status as Succes to notify the user

###### 18.9.1.13 --

| CodeBeamer reference: | APAVPCSR-3416239 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (3)

| APAVPC-3415554 | Selection of Control mode for V_Comfort Operation |
| --- | --- |
| APAVPC-3415559 | Pre-Maneuver Condition Check and Status Communication |
| APAVPC-3415584 | Park-In Maneuver Initiation: Control mode change |

Downstream References (1)

| APASAD-3412216 | Sequence Diagram |
| --- | --- |

No Associations

Description

When
VPC_Status is Path-planning and selection 
OR
VPC_Status is Slot Display and Auto-Maneuvering
AND
C-App_Maneuver_Command is True
OR
(MHU_Maneuver_Command is True AND DMS_DriverOccupancy is True)
Then
V-Parking Comfort shall check for the preconditions for Auto-Maneuvering

##### 18.9.2 Pre- conditions for Auto- Maneuvering:

| CodeBeamer reference: | APAVPCSR-3415841 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

###### 18.9.2.1 Descrption

| CodeBeamer reference: | APAVPCSR-3416240 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

Auto Maneuvering possible only when all pre conditions are satisfied.

###### 18.9.2.2 Input Signals

| CodeBeamer reference: | APAVPCSR-3416241 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

IDB_BrakePedal_State
VCU_Gear_State 
VCU_Veh_Spd 
Sensor_Input_Status
Camera_Input_Status
BCM_STAT_BonnetAjar
CMS_Occupancy
BCM_Trailer_Attachment_Status
BCM_Windows_Lock_Status
BMS_Battery_SoC
TPMS_Tire_Pressure
BCM_Doors_Lock_Status
BCM_STAT_TrunkAjar
BCM_STAT_Mirror
MHU_Maneuver_Command
DMS_DriverOccupancy
VCU_Veh_Spd
ACU_STAT_Airbag
ACM_Seat_Belt_Status_Driver
IDB_BrakePedal_State

###### 18.9.2.3 Output Signals

| CodeBeamer reference: | APAVPCSR-3416242 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

Upstream References (12)

| APAVPC-3415594 | Driver Proximity Requirement for Remote Control |
| --- | --- |
| APAVPC-3415584 | Park-In Maneuver Initiation: Control mode change |
| APAVPC-3415585 | Status Updates during Park-in maneuver: Remote |
| APAVPC-3415607 | Real-Time Trajectory and Vehicle Status Display - Remote Park-Out |
| APAVPC-3415554 | Selection of Control mode for V_Comfort Operation |
| APAVPC-3415559 | Pre-Maneuver Condition Check and Status Communication |
| APAVPC-3415545 | Remote Mode Selection: Occupancy detection |
| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| APAVPC-3415617 | Remote Interface Connectivity |
| AVPCSR-3416567 | -- |
| AVPCSR-3416574 | -- |
| AVPCSR-3416566 | -- |

No Downstream References

No Associations

Description

VPC_Status
VPC_User_Comm_dev
VPC_C-App_Comm_Status
VPC_Maneuver_Precond

###### 18.9.2.4 --

| CodeBeamer reference: | APAVPCSR-3416243 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415559 | Pre-Maneuver Condition Check and Status Communication |
| --- | --- |

Downstream References (1)

| APASAD-3412216 | Sequence Diagram |
| --- | --- |

No Associations

Description

When
VPC_Precond is True
AND
User_Comm_dev is C-app
AND
Brake Pedal and steering is released
AND
gear_selection is Park
Then
V Parking Comfort System shall set VPC_Maneuver_precond as True

###### 18.9.2.5 VF_PF_SYSRS_1597

| CodeBeamer reference: | APAVPCSR-3416244 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

Upstream References (1)

| APAVPC-3415559 | Pre-Maneuver Condition Check and Status Communication |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path-planning and selection 
OR
VPC_Status is Slot Display and Auto-Maneuvering
AND
(MHU_Maneuver_Command is True AND DMS_DriverOccupancy is True) 
AND
VCU_Veh_Spd =0
AND
Sensor_Input_Status is True
AND
Camera_Input_Status is True
AND
BCM_STAT_BonnetAjar is closed
AND
BMS_Battery_SoC > [Battery_SoC_THD]
AND
BCM_Trailer_Attachment_Status is False
AND
ACU_STAT_Airbag is Active
AND
TPMS_Tire_Pressure > [MIN_TIRE_PRESSURE]
AND
BCM_Windows_Lock_Status is locked
AND 
ACM_Seat_Belt_Status_Driver is Buckled
AND
IDB_BrakePedal_State is Released 
AND
VCU_Gear_State is Park
AND
BCM_Doors_Lock_Status is locked 
AND
BCM_STAT_TrunkAjar is closed
AND
ACU_Seat_Adj_Status is Inactive
AND
BCM_STAT_Mirror is unfolded
Then
The V-Parking System shall set VPC_Maneuver_Precond as True

Note:
Maneuvering precondition for when the driver is controlling the maneuver from inside the vehicle.

##### 18.9.3 Path-Maneuvering/moving:

| CodeBeamer reference: | APAVPCSR-3415842 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

###### 18.9.3.1 Descrption

| CodeBeamer reference: | APAVPCSR-3416245 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

Feature automatically guides the vehicle along a planned path at low speed, ensuring safe park-in or park-out with obstacle detection and suspension on safety triggers.

###### 18.9.3.2 Input Signals

| CodeBeamer reference: | APAVPCSR-3416246 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

MHU_Maneuver_Command
C-App_Maneuver_Command
Keyfob_Maneuver_Command 
DMS_DriverOccupancy
BCM_Doors_Lock_Status
MHU_Park_Type_Req
C-App_Park_Type_Req
IDB_Ctrl_Status
EPB_Ctrl_Status
EPS_Steering_Ctrl_Status
VCU_Gear_Ctrl_Status
VCU_Throttle_Ctrl_Status
IDB_BrakePedal_State
IDB_Ctrl_Status
DMS_Occupancy
BCM_Doors_Lock_Status
BCM_STAT_Mirror

###### 18.9.3.3 Output Signals

| CodeBeamer reference: | APAVPCSR-3416247 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

Upstream References (2)

| APAVPC-3415559 | Pre-Maneuver Condition Check and Status Communication |
| --- | --- |
| AVPCSR-3416627 | -- |

No Downstream References

No Associations

Description

VPC_Status
VPC_EPB_Ctrl_Req
VPC_EPS_Steering_Ctrl_Req 
VPC_IDB_Ctrl_Req
VPC_VCU_Gear_Ctrl_Req
VPC_VCU_Throttle_Ctrl_Req
VPC_IDB_Cmd
VPC_EPB_Cmd 
VPC_EPS_Steering_Cmd
VPC_VCU_Throttle_Cmd
VPC_VCU_Gear_Cmd
VPC_EPB_Brake_Req 
Reason_for_suspend
VPC_User_Comm_dev
VPC_Keyfob_Communication_Status
VPC_C-App_Communication_Status
VPC_Maneuver_Precond
VPC_Keyfob_Commu_Status 
VPC_C-App_Commu_Status

###### 18.9.3.4 Internal Signals

| CodeBeamer reference: | APAVPCSR-3416248 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

Obstcl_Identified
Alternate_Path_Planning
Movable_object_in_path

###### 18.9.3.5 --

| CodeBeamer reference: | APAVPCSR-3416249 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415584 | Park-In Maneuver Initiation: Control mode change |
| --- | --- |

Downstream References (1)

| APASAD-3412216 | Sequence Diagram |
| --- | --- |

No Associations

Description

When
VPC_Status is Path Planning and Selection
AND
(MHU_Maneuver_Command is True AND DMS_DriverOccupancy is True)
OR 
C-App_Maneuver_Command is True 
OR 
Keyfob_Maneuver_Command is True
AND
VPC_Maneuver_Precond is true
Then
V-Parking Comfort Select shall set VPC_Status as Path-Maneuvering/moving

###### 18.9.3.6 VF_PF_SYSRS_1598

| CodeBeamer reference: | APAVPCSR-3416250 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415564 | Low-speed forward navigation Execution |
| --- | --- |

No Downstream References

No Associations

Description

When 
VPC_Status is Status is Path-Maneuvering/moving
AND
VPC_Maneuver_Precond is True
AND
MHU_Park_Type_Req is Park-out
OR
C-App_Park_Type_Req is Park-out
Then
The V-Parking Comfort shall send VPC_EPB_Ctrl_Req as True to request control of the park brake

###### 18.9.3.7 VF_PF_SYSRS_1599

| CodeBeamer reference: | APAVPCSR-3416251 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415564 | Low-speed forward navigation Execution |
| --- | --- |

No Downstream References

No Associations

Description

When 
VPC_Status is Status is Path-Maneuvering/moving
AND
VPC_Maneuver_Precond is True
AND
MHU_Park_Type_Req is Park-out
OR
C-App_Park_Type_Req is Park-out
Then
The V-Parking Comfort shall send VPC_EPS_Steering_Ctrl_Req as True to request control of the Steering

###### 18.9.3.8 VF_PF_SYSRS_1600

| CodeBeamer reference: | APAVPCSR-3416252 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415564 | Low-speed forward navigation Execution |
| --- | --- |

No Downstream References

No Associations

Description

When 
VPC_Status is Status is Path-Maneuvering/moving
AND
VPC_Maneuver_Precond is True
AND
MHU_Park_Type_Req is Park-out
OR
C-App_Park_Type_Req is Park-out
Then
The V-Parking Comfort shall send VPC_IDB_Ctrl_Req (True) to request control of the brake

###### 18.9.3.9 VF_PF_SYSRS_1601

| CodeBeamer reference: | APAVPCSR-3416253 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415564 | Low-speed forward navigation Execution |
| --- | --- |

No Downstream References

No Associations

Description

When 
VPC_Status is Path-Maneuvering/moving
AND
VPC_Maneuver_Precond is True
AND
MHU_Park_Type_Req is Park-out
OR
C-App_Park_Type_Req is Park-out
Then
The V-Parking Comfort shall send VPC_VCU_Gear_Ctrl_Req (True) to request control of the gear selector

###### 18.9.3.10 VF_PF_SYSRS_1602

| CodeBeamer reference: | APAVPCSR-3416254 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415564 | Low-speed forward navigation Execution |
| --- | --- |

No Downstream References

No Associations

Description

When 
VPC_Status is Path-Maneuvering/moving
AND
VPC_Maneuver_Precond is True
AND
MHU_Park_Type_Req is Park-out
OR
C-App_Park_Type_Req is Park-out
Then
The V-Parking Comfort shall send VPC_VCU_Throttle_Ctrl_Req (True) to request control of the throttle

###### 18.9.3.11 VF_PF_SYSRS_1603

| CodeBeamer reference: | APAVPCSR-3416255 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| AVPCSR-3417042 | -- |
| --- | --- |
| APAVPC-3415564 | Low-speed forward navigation Execution |

No Downstream References

No Associations

Description

When 
VPC_Status is Status is Path-Maneuvering/moving
AND
VPC_Maneuver_Precond is True
AND
IDB_Ctrl_Status is System Control
AND 
EPB_Ctrl_Status is System Control
AND
EPS_Steering_Ctrl_Status is System Control
AND
VCU_Throttle_Ctrl_Status is System Control
AND
VCU_Gear_Ctrl_Status is System Control
Then
The V-Parking Comfort shall send VPC_IDB_Cmd to control brake

###### 18.9.3.12 VF_PF_SYSRS_1604

| CodeBeamer reference: | APAVPCSR-3416256 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415564 | Low-speed forward navigation Execution |
| --- | --- |

No Downstream References

No Associations

Description

When 
VPC_Status is Path-Maneuvering/moving
AND
VPC_Maneuver_Precond is True
AND
IDB_Ctrl_Status is System Control
AND 
EPB_Ctrl_Status is System Control
AND
EPS_Steering_Ctrl_Status is System Control
AND
VCU_Throttle_Ctrl_Status is System Control
AND
VCU_Gear_Ctrl_Status is System Control
Then
The V-Parking Comfort shall send VPC_EPB_Cmd to control Parking brake

###### 18.9.3.13 VF_PF_SYSRS_1605

| CodeBeamer reference: | APAVPCSR-3416257 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415564 | Low-speed forward navigation Execution |
| --- | --- |
| AVPCSR-3417042 | -- |

No Downstream References

No Associations

Description

When 
VPC_Status is Path-Maneuvering/moving
AND
VPC_Maneuver_Precond is True
AND
IDB_Ctrl_Status is System Control
AND 
EPB_Ctrl_Status is System Control
AND
EPS_Steering_Ctrl_Status is System Control
AND
VCU_Throttle_Ctrl_Status is System Control
AND
VCU_Gear_Ctrl_Status is System Control
Then
The V-Parking Comfort shall send VPC_EPS_Steering_Cmd to EPS to Control Steering

###### 18.9.3.14 VF_PF_SYSRS_1606

| CodeBeamer reference: | APAVPCSR-3416258 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415564 | Low-speed forward navigation Execution |
| --- | --- |
| AVPCSR-3417042 | -- |

No Downstream References

No Associations

Description

When 
VPC_Status is Path-Maneuvering/moving
AND
VPC_Maneuver_Precond is True
AND
IDB_Ctrl_Status is System Control
AND 
EPB_Ctrl_Status is System Control
AND
EPS_Steering_Ctrl_Status is System Control
AND
VCU_Throttle_Ctrl_Status is System Control
AND
VCU_Gear_Ctrl_Status is Sytsem Control
Then
The V-Parking Comfort shall send VPC_VCU_Throttle_Cmd to VCU to control throttle

###### 18.9.3.15 VF_PF_SYSRS_1607

| CodeBeamer reference: | APAVPCSR-3416259 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415564 | Low-speed forward navigation Execution |
| --- | --- |
| AVPCSR-3417042 | -- |

No Downstream References

No Associations

Description

When 
VPC_Status is Path-Maneuvering/moving
AND
VPC_Maneuver_Precond is True
AND
IDB_Ctrl_Status is System Control
AND 
EPB_Ctrl_Status is System Control
AND
EPS_Steering_Ctrl_Status is System Control
AND
VCU_Throttle_Ctrl_Status is System Control
AND
VCU_Gear_Ctrl_Status is System Control
Then
The V-Parking Comfort shall send VPC_VCU_Gear_Cmd to VCU to Control gear selector

###### 18.9.3.16 --

| CodeBeamer reference: | APAVPCSR-3416260 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415584 | Park-In Maneuver Initiation: Control mode change |
| --- | --- |
| APAVPC-3415562 | Mid-maneuver Condition Check and Status Communication |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When
VPC_Status is Path-Maneuvering/moving
AND
(MHU_Maneuver_Command is True AND DMS_DriverOccupancy is True) 
OR 
C-App_Maneuver_Command is True 
OR 
Keyfob_Maneuver_Command is True
Then
V-Parking Comfort shall move in Trace_Path coordinate values at a speed less than or equal to MAX_AUTO_MANEUVERING_SPEED [10kmph +or- 2kmph(tolerance)] / 

Note: 
C-App/Keyfob Command need to be continously active for maneuvering.

###### 18.9.3.17 --

| CodeBeamer reference: | APAVPCSR-3416261 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415565 | Remote Mode Selection: Continuous Engagement via Mobile App |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When
VPC_Status is Path maneuvering/moving
Then
APA shall scan surrounding actively to detect obstacle

###### 18.9.3.18 --

| CodeBeamer reference: | APAVPCSR-3416262 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

When
VPC_Status is Suspend
Then
V-Parking Comfort shall set VPC_EPB_Brake_Req as True to bring vehicle to stationary.

###### 18.9.3.19 VF_PF_SYSRS_1608

| CodeBeamer reference: | APAVPCSR-3416263 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Suspend
Then
V-Parking Comfort shall send Reason_for_suspend to VPC_User_Comm_dev (MHU/ C-App)

###### 18.9.3.20 --

| CodeBeamer reference: | APAVPCSR-3416264 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |
| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |

No Downstream References

No Associations

Description

When
VPC_Status is Suspend for more than MAX_SUSPEND_TIME
Then
V-Parking Comfort shall set VPC_Status as Abort

###### 18.9.3.21 --

| CodeBeamer reference: | APAVPCSR-3416265 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (1)

| APASAD-3412216 | Sequence Diagram |
| --- | --- |

No Associations

Description

When
VPC_Status is Path-Maneuvering/moving
AND
VPC_Keyfob_Communication_Status is Failed 
OR 
VPC_C-App_Communication_Status is Failed
OR
DMS_Occupancy is False
Then
V-Parking Comfort shall set VPC_Status as Suspend

###### 18.9.3.22 --

| CodeBeamer reference: | APAVPCSR-3416266 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When 
VPC_Status is Path-Maneuvering/moving
AND
IDB_BrakePedal_State is Applied 
AND 
IDB_Ctrl_Status is Driver Control
Then 
V-Parking Comfort shall set VPC_Status as Suspend.

###### 18.9.3.23 VF_PF_SYSRS_1609

| CodeBeamer reference: | APAVPCSR-3416267 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When 
VPC_Status is Path-Maneuvering/moving
AND
IDB_BrakePedal_State is Applied 
AND 
IDB_Ctrl_Status is Driver Control
Then 
V-Parking Comfort shall set Reason_for_suspend as Req_Recover_Press_brake (4). .

###### 18.9.3.24 --

| CodeBeamer reference: | APAVPCSR-3416268 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When 
VPC_Status is Path-Maneuvering/moving
AND
VPC_Maneuver_precond is False
Then 
V-Parking Comfort shall set VPC_Status as Suspend.

###### 18.9.3.25 --

| CodeBeamer reference: | APAVPCSR-3416269 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When
VPC_Status is Path-Maneuvering/moving
AND
Obstcl_Identified is True in the target_trajectory
AND
Alternate_Path_Planning is False 
Then
V-Parking Comfort shall set VPC_Status as Suspend.

###### 18.9.3.26 VF_PF_SYSRS_1610

| CodeBeamer reference: | APAVPCSR-3416270 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path-Maneuvering/moving
AND
Obstcl_Identified is True in the target_trajectory
Then
V-Parking Comfort shall inform about the object to the driver

###### 18.9.3.27 --

| CodeBeamer reference: | APAVPCSR-3416271 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When 
VPC_Status is Path-Maneuvering/moving
AND
Driver release button, VPC_Keyfob_Commu_Status is False OR VPC_C-App_Commu_Status is False
Then 
V-Parking Comfort shall set VPC_Status as Suspend.

###### 18.9.3.28 --

| CodeBeamer reference: | APAVPCSR-3416272 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
Movable_object_in_path is true
AND
Alternate_Path_Planning is False
Then
V-Parking Comfort shall set Reason_for_suspend as Req_Recover_FoundObstacleInTrajectory

###### 18.9.3.29 VF_PF_SYSRS_1611

| CodeBeamer reference: | APAVPCSR-3416273 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
Movable_object_in_path is true
AND
Alternate_Path_Planning is False
Then
V-Parking Comfort shall set VPC_Status as suspend

###### 18.9.3.30 --

| CodeBeamer reference: | APAVPCSR-3416274 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
BCM_Doors_Lock_Status is Unlocked
Then
V-Parking Comfort shall set Reason_for_suspend as Req_Recover_Doors_unlock

###### 18.9.3.31 VF_PF_SYSRS_1612

| CodeBeamer reference: | APAVPCSR-3416275 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
BCM_Doors_Lock_Status is Unlocked
Then
V-Parking Comfort shall set VPC_Status as suspend

###### 18.9.3.32 --

| CodeBeamer reference: | APAVPCSR-3416276 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
BCM_STAT_Mirror is folded
Then
V-Parking Comfort shall set VPC_Status as suspend

###### 18.9.3.33 --

| CodeBeamer reference: | APAVPCSR-3416277 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
BCM_STAT_DoorAjarFL OR BCM_STAT_DoorAjarFR OR BCM_STAT_DoorAjarRL OR BCM_STAT_DoorAjarRR is Open
Then
V-Parking Comfort shall set Reason_for_suspend as Req_Recover_Close_Doors (3)

###### 18.9.3.34 VF_PF_SYSRS_1613

| CodeBeamer reference: | APAVPCSR-3416278 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
BCM_STAT_DoorAjarFL OR BCM_STAT_DoorAjarFR OR BCM_STAT_DoorAjarRL OR BCM_STAT_DoorAjarRR is open
Then
V-Parking Comfort shall set VPC_Status as suspend

###### 18.9.3.35 --

| CodeBeamer reference: | APAVPCSR-3416279 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
DMS_button_released is true
Then
V-Parking Comfort shall set Reason_for_suspend as Req_Recover_DMS_Release (5)

###### 18.9.3.36 VF_PF_SYSRS_1614

| CodeBeamer reference: | APAVPCSR-3416280 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
DMS_button_released is true
Then
V-Parking Comfort shall set VPC_Status as suspend

###### 18.9.3.37 --

| CodeBeamer reference: | APAVPCSR-3416281 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
BCM_STAT_TrunkAjar is open
Then
V-Parking Comfort shall set VPC_Status as suspend

###### 18.9.3.38 --

| CodeBeamer reference: | APAVPCSR-3416282 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
Mode_select is true
Then
V-Parking Comfort shall set Reason_for_suspend as Req_Mode Select (1)

###### 18.9.3.39 VF_PF_SYSRS_1615

| CodeBeamer reference: | APAVPCSR-3416283 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
Mode_select is true
Then
V-Parking Comfort shall set VPC_Status as suspend

###### 18.9.3.40 --

| CodeBeamer reference: | APAVPCSR-3416284 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
Pull_out_direction_selected is false
Then
V-Parking Comfort shall set Reason_for_suspend as Req_Select_Pull_out_Direction (D)

###### 18.9.3.41 VF_PF_SYSRS_1616

| CodeBeamer reference: | APAVPCSR-3416285 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
Pull_out_direction_selected is false
Then
V-Parking Comfort shall set VPC_Status as suspend

###### 18.9.3.42 --

| CodeBeamer reference: | APAVPCSR-3416286 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (1)

| APASAD-3412216 | Sequence Diagram |
| --- | --- |

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
C-App_Maneuver_Command is False
OR
Keyfob_Maneuver_Command is False
Then
V-Parking Comfort shall set Reason_for_suspend as Req_Press_ParkingSwitch (E)

###### 18.9.3.43 VF_PF_SYSRS_1617

| CodeBeamer reference: | APAVPCSR-3416287 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
C-App_Maneuver_Command is False
OR
Keyfob_Maneuver_Command is False
Then
V-Parking Comfort shall set VPC_Status as suspend

###### 18.9.3.44 --

| CodeBeamer reference: | APAVPCSR-3416288 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
C-App_Maneuver_Command is True
OR
Keyfob_Maneuver_Command is True
AND
CMS_Occupancy is true
Then
V-Parking Comfort shall set Reason_for_suspend as Req_Occu_Vehicle (11)

###### 18.9.3.45 VF_PF_SYSRS_1618

| CodeBeamer reference: | APAVPCSR-3416289 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
C-App_Maneuver_Command is True
OR
Keyfob_Maneuver_Command is True
AND
CMS_Occupancy is true
Then
V-Parking Comfort shall set VPC_Status as suspend

###### 18.9.3.46 --

| CodeBeamer reference: | APAVPCSR-3416290 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (1)

| APASAD-3412216 | Sequence Diagram |
| --- | --- |

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
Remote_device_connected is false
Then
V-Parking Comfort shall set Reason_for_suspend as Req_Connect_RD (13)

###### 18.9.3.47 VF_PF_SYSRS_1619

| CodeBeamer reference: | APAVPCSR-3416291 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
Remote_device_connected is false
Then
V-Parking Comfort shall set VPC_Status as suspend

###### 18.9.3.48 --

| CodeBeamer reference: | APAVPCSR-3416292 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
VPC_Paused is True
Then
V-Parking Comfort shall set VPC_Status as suspend

###### 18.9.3.49 --

| CodeBeamer reference: | APAVPCSR-3416293 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (1)

| APASAD-3412216 | Sequence Diagram |
| --- | --- |

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
Slot_searching is false
Then
V-Parking Comfort shall set Reason_for_suspend as Req_SearchingProcess (12)

###### 18.9.3.50 VF_PF_SYSRS_1620

| CodeBeamer reference: | APAVPCSR-3416294 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415625 | System states in Drive mode |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
Slot_searching is false
Then
V-Parking Comfort shall set VPC_Status as suspend

###### 18.9.3.51 --

| CodeBeamer reference: | APAVPCSR-3416295 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415624 | Pause Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
EPB_Brake_State is Applied
Then
V-Parking Comfort shall set Reason_for_suspend as Req_Recover_Press_brake (4)

###### 18.9.3.52 VF_PF_SYSRS_1621

| CodeBeamer reference: | APAVPCSR-3416296 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving
AND
EPB_Brake_State is Applied
Then
V-Parking Comfort shall set VPC_Status as suspend

#### 18.10 Finished:

| CodeBeamer reference: | APAVPCSR-3415727 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.10.1 Descrption

| CodeBeamer reference: | APAVPCSR-3415843 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

VPC ensures the vehicle is properly aligned and centered in the parking slot, applies final safety actions, and completes the maneuver only when all conditions are met.

##### 18.10.2 Input Signals

| CodeBeamer reference: | APAVPCSR-3415844 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

MHU_Maneuver_Command
C-App_Maneuver_Command 
Keyfob_Maneuver_Command
DMS_Occupancy
BCM_Doors_Lock_Status
MHU_Selected_ Slot_ID

##### 18.10.3 Output Signals

| CodeBeamer reference: | APAVPCSR-3415845 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

VPC_EPS_Steering_Cmd
VPC_EPB_Cmd
VPC_VCU_Gear_Cmd

##### 18.10.4 --

| CodeBeamer reference: | APAVPCSR-3415846 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415589 | Park-In Completion Activities: Remote driver |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When 
VPC_Status is Finished
AND
TCU_Current_Veh_Posn equals Target_Coordinate.
AND
Cancel_pressed is false
Then
VPC_EPS_Steering_Cmd as Center to return steering wheel to neutral

##### 18.10.5 VF_PF_SYSRS_1622

| CodeBeamer reference: | APAVPCSR-3415847 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (3)

| APAVPC-3415589 | Park-In Completion Activities: Remote driver |
| --- | --- |
| AVPCSR-3416558 | -- |
| AVPCSR-3416569 | -- |

No Downstream References

No Associations

Description

When 
VPC_Status is Finished
AND
TCU_Current_Veh_Posn equals Target_Coordinate
AND
Cancel_pressed is false
Then
set VPC_EPB_Cmd as Applied

##### 18.10.6 VF_PF_SYSRS_1623

| CodeBeamer reference: | APAVPCSR-3415848 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (3)

| AVPCSR-3416569 | -- |
| --- | --- |
| AVPCSR-3416558 | -- |
| APAVPC-3415589 | Park-In Completion Activities: Remote driver |

No Downstream References

No Associations

Description

When 
VPC_Status is Finished
AND
TCU_Current_Veh_Posn equals Target_Coordinate
AND
Cancel_pressed is false
Then
VPC_VCU_Gear_Cmd as Park

##### 18.10.7 VF_PF_SYSRS_1624

| CodeBeamer reference: | APAVPCSR-3415849 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (3)

| AVPCSR-3416569 | -- |
| --- | --- |
| AVPCSR-3416558 | -- |
| APAVPC-3415589 | Park-In Completion Activities: Remote driver |

No Downstream References

No Associations

Description

When 
VPC_Status is Finished
AND
TCU_Current_Veh_Posn equals Target_Coordinate
Then
set VPC_OccupancyCheck_Req as True

##### 18.10.8 VF_PF_SYSRS_1625

| CodeBeamer reference: | APAVPCSR-3415850 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415589 | Park-In Completion Activities: Remote driver |
| --- | --- |

No Downstream References

No Associations

Description

When 
VPC_Status is Finished
AND
TCU_Current_Veh_Posn equals Target_Coordinate
Then
V-parking Comfort shall set BCM_Doors_Lock_Status as True

##### 18.10.9 --

| CodeBeamer reference: | APAVPCSR-3415851 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415589 | Park-In Completion Activities: Remote driver |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When
VPC_Status is Finished
AND
Park_Type is Park in
AND
MHU_Maneuver_Command is True
AND
VPC_EPS_Steering_Cmd is Center 
AND
VPC_EPB_Cmd is Applied
AND 
VPC_VCU_Gear_Cmd is Park
AND 
VPC_BCM_Light_Status is OFF
AND
VPC_BCM_Window_Status is Closed
AND
BCM_Doors_Lock_Status as True
Then
V-Parking Comfort shall set VPC_Park in_Finished_Status as success

##### 18.10.10 VF_PF_SYSRS_1626

| CodeBeamer reference: | APAVPCSR-3415852 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (3)

| APAVPC-3415589 | Park-In Completion Activities: Remote driver |
| --- | --- |
| AVPCSR-3416551 | -- |
| AVPCSR-3416558 | -- |

No Downstream References

No Associations

Description

When
VPC_Status is Finished
AND
Park_Type is Park in
AND
C-App_Maneuver_Command is True 
OR 
Keyfob_Maneuver_Command is True
AND
VPC_EPS_Steering_Cmd as Center 
AND
VPC_EPB_Cmd as Applied
AND 
VPC_VCU_Gear_Cmd as Park
AND 
VPC_BCM_Light_Status is OFF
AND
DMS_Occupancy is False
AND
VPC_BCM_Window_Status is Closed
AND
BCM_Doors_Lock_Status is Lock
Then
V-Parking Comfort shall set VPC_Park in_Finished_Status as success

##### 18.10.11 --

| CodeBeamer reference: | APAVPCSR-3415853 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415587 | Park-In Completion: Alignment Check |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When 
VPC_Status is Finished,
Then 
V-Select Comfort shall ensure the vehicle is centered within the parking slot, with equal distance on both sides.

##### 18.10.12 --

| CodeBeamer reference: | APAVPCSR-3415854 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415587 | Park-In Completion: Alignment Check |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When 
VPC_Status is Finished,
AND
Park_type is Park in
AND
Slot_markings_identified is true
Then 
V-Parking Comfort shall ensure vehicle positioned the within the marked boundaries of the selected slot (MHU_Selected_ Slot_ID).

##### 18.10.13 --

| CodeBeamer reference: | APAVPCSR-3415855 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415587 | Park-In Completion: Alignment Check |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When 
VPC_Status is Finished
AND
Park_type is Park in
AND
Slot_markings_identified is false
Then 
V-Parking Comfort shall ensure the vehicle oriented with selected slot (MHU_Selected_ Slot_ID) to aligned with adjacent parked vehicle

##### 18.10.14 --

| CodeBeamer reference: | APAVPCSR-3415856 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415587 | Park-In Completion: Alignment Check |
| --- | --- |

Downstream References (1)

| APASAD-3412216 | Sequence Diagram |
| --- | --- |

No Associations

Description

When 
VPC_Status is Finished
AND
Park_type is Park in
AND
VPC_Park in_Finished_Status is False
AND
EPS_Steering_Override is True
OR
VCU_Throttle_Override is True
Then 
V-Parking Comfort shall set VPC_status as abort and inform driver

##### 18.10.15 VF_PF_SYSRS_1627

| CodeBeamer reference: | APAVPCSR-3415857 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (4)

| AVPCSR-3416559 | -- |
| --- | --- |
| AVPCSR-3417045 | -- |
| AVPCSR-3417046 | -- |
| APAVPC-3415587 | Park-In Completion: Alignment Check |

No Downstream References

No Associations

Description

When 
VPC_Status is Finished
AND
Park_type is Park in
AND
VPC_Park in_Finished_Status is False
AND
EPS_Steering_Override is True
OR
VCU_Throttle_Override is True
Then
The V-Parking Comfort shall send VPC_VCU_Gear_Ctrl_Req as False to VCU --> to release control of the gear selector

##### 18.10.16 VF_PF_SYSRS_1628

| CodeBeamer reference: | APAVPCSR-3415858 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (4)

| APAVPC-3415587 | Park-In Completion: Alignment Check |
| --- | --- |
| AVPCSR-3416559 | -- |
| AVPCSR-3417045 | -- |
| AVPCSR-3417046 | -- |

No Downstream References

No Associations

Description

When 
VPC_Status is Finished
AND
Park_type is Park in
AND
VPC_Park in_Finished_Status is False
AND
EPS_Steering_Override is True
OR
VCU_Throttle_Override is True
Then
V-Parking Comfort shall send VPC_EPB_Ctrl_Req as False to EPB --> to release the control of park brake

##### 18.10.17 VF_PF_SYSRS_1629

| CodeBeamer reference: | APAVPCSR-3415859 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (4)

| APAVPC-3415587 | Park-In Completion: Alignment Check |
| --- | --- |
| AVPCSR-3416559 | -- |
| AVPCSR-3417046 | -- |
| AVPCSR-3417045 | -- |

No Downstream References

No Associations

Description

When 
VPC_Status is Finished
AND
Park_type is Park in
AND
VPC_Park in_Finished_Status is False
AND
EPS_Steering_Override is True
OR
VCU_Throttle_Override is True
Then
The V-Parking Comfort shall send VPC_IDB_Ctrl_Req as False to IDB --> to release the control of the brake

##### 18.10.18 VF_PF_SYSRS_1630

| CodeBeamer reference: | APAVPCSR-3415860 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (4)

| AVPCSR-3416559 | -- |
| --- | --- |
| APAVPC-3415587 | Park-In Completion: Alignment Check |
| AVPCSR-3417045 | -- |
| AVPCSR-3417046 | -- |

No Downstream References

No Associations

Description

When 
VPC_Status is Finished
AND
Park_type is Park in
AND
VPC_Park in_Finished_Status is False
AND
EPS_Steering_Override is True
OR
VCU_Throttle_Override is True
Then
The V-Parking Comfort shall send VPC_EPS_Steering_Ctrl_Req as False to EPS --> to release the control of the steering

##### 18.10.19 VF_PF_SYSRS_1631

| CodeBeamer reference: | APAVPCSR-3415861 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (4)

| AVPCSR-3417046 | -- |
| --- | --- |
| AVPCSR-3417045 | -- |
| AVPCSR-3416559 | -- |
| APAVPC-3415587 | Park-In Completion: Alignment Check |

No Downstream References

No Associations

Description

When 
VPC_Status is Finished
AND
Park_type is Park in
AND
VPC_Park in_Finished_Status is False
AND
EPS_Steering_Override is True
OR
VCU_Throttle_Override is True
Then
The V-Parking Comfort shall send VPC_VCU_Throttle_Ctrl_Req as False to VCU --> to release control of throttle

#### 18.11 Completed:

| CodeBeamer reference: | APAVPCSR-3415728 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 18.12 Abort/ Safe stop:

| CodeBeamer reference: | APAVPCSR-3415729 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.12.1 Description

| CodeBeamer reference: | APAVPCSR-3415862 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

VPC_Status is aborted if activation conditions are not met or a system fault is detected, ensuring safe termination of the maneuver.

##### 18.12.2 Input Signals

| CodeBeamer reference: | APAVPCSR-3415863 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

MHU_VPC_Cancelled_Req
Remote_device_engagement 
Manual_override
Sensor_malfunction 
Software_error
Poor_visibility
Number_of_moves
Low_battery_conditions

##### 18.12.3 Output Signals

| CodeBeamer reference: | APAVPCSR-3415864 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

VPC_EPB_Ctrl_Req
VPC_Status
APA_status
APA_system_failure

##### 18.12.4 Internal Signals

| CodeBeamer reference: | APAVPCSR-3415865 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

Upstream References (1)

| APAVPC-3415586 | Park-In Alerts for Driver and Surroundings |
| --- | --- |

No Downstream References

No Associations

Description

Vehicle_speed

##### 18.12.5 --

| CodeBeamer reference: | APAVPCSR-3415866 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415586 | Park-In Alerts for Driver and Surroundings |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

When
VPC_Status is Abort
Then
V-Parking System shall set Reason_for_abort in C-App for displying the reason for abort.

##### 18.12.6 VF_PF_SYSRS_1632

| CodeBeamer reference: | APAVPCSR-3415867 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415586 | Park-In Alerts for Driver and Surroundings |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Abort
Then
V-Parking Comfort shall set VPC_EPB_Ctrl_Req

##### 18.12.7 VF_PF_SYSRS_1633

| CodeBeamer reference: | APAVPCSR-3415868 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415586 | Park-In Alerts for Driver and Surroundings |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Abort
Then
The V-Parking shall send IDB_BrakePedal_State as Apply to IDB --> to bring the vehicle to a stop

##### 18.12.8 VF_PF_SYSRS_1634

| CodeBeamer reference: | APAVPCSR-3415869 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415586 | Park-In Alerts for Driver and Surroundings |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Abort
Then
V-Parking Comfort shall send EPB_Brake_State as Apply to EPB --> to bring vehicle to a stop

##### 18.12.9 VF_PF_SYSRS_1635

| CodeBeamer reference: | APAVPCSR-3415870 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415586 | Park-In Alerts for Driver and Surroundings |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Abort
AND
IDB_BrakePedal_State is Applied 
AND
EPB_Brake_State is Applied
AND
VCU_Veh_Spd is 0 
Then
V-Parking Comfort shall send VPC_EPB_Ctrl_Req as False to EPB --> to release the control of park brake

##### 18.12.10 VF_PF_SYSRS_1636

| CodeBeamer reference: | APAVPCSR-3415871 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415586 | Park-In Alerts for Driver and Surroundings |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Abort
AND
IDB_BrakePedal_State is Applied 
AND
EPB_Brake_State is Applied
AND
VCU_Veh_Spd is 0 
Then
The V-Parking Comfort shall send VPC_IDB_Ctrl_Req as False to IDB --> to release the control of the brake

##### 18.12.11 VF_PF_SYSRS_1637

| CodeBeamer reference: | APAVPCSR-3415872 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

When
VPC_Status is Abort
AND
IDB_BrakePedal_State is Applied 
AND
EPB_Brake_State is Applied 
AND
VCU_Veh_Spd is 0 
Then
The V-Parking Comfort shall send VPC_EPS_Steering_Ctrl_Req as False to EPS --> to release the control of the steering

##### 18.12.12 VF_PF_SYSRS_1638

| CodeBeamer reference: | APAVPCSR-3415873 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

When
VPC_Status is Abort
AND
IDB_BrakePedal_State is Applied 
AND
EPB_Brake_State is Applied
AND
VCU_Veh_Spd is 0 
Then
The V-Parking Comfort shall send VPC_VCU_Throttle_Ctrl_Req as False to VCU --> to release control of throttle

##### 18.12.13 VF_PF_SYSRS_1639

| CodeBeamer reference: | APAVPCSR-3415874 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

When
VPC_Status is Abort
AND
IDB_BrakePedal_State is Applied 
AND
EPB_Brake_State is Applied
AND
VCU_Veh_Spd is 0 
Then
The V-Parking Comfort shall send VPC_VCU_Gear_Ctrl_Req as False to VCU --> to release control of the gear selector

##### 18.12.14 --

| CodeBeamer reference: | APAVPCSR-3415875 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415586 | Park-In Alerts for Driver and Surroundings |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When 
APA_status is APA_Enabled
AND
VPC_Status is other than Reset
AND
The user disables APA 
Then 
V-parking Comfort shall set MHU_VPC_Cancelled_Req as true

##### 18.12.15 VF_PF_SYSRS_1640

| CodeBeamer reference: | APAVPCSR-3415876 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

Upstream References (1)

| APAVPC-3415586 | Park-In Alerts for Driver and Surroundings |
| --- | --- |

No Downstream References

No Associations

Description

When 
APA_status is APA_Enabled
AND
VPC_Status is other than Reset
AND
The user disables APA 
Then 
APA system shall set APA_status as APA_Disabled

##### 18.12.16 VF_PF_SYSRS_1641

| CodeBeamer reference: | APAVPCSR-3415877 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

Upstream References (1)

| APAVPC-3415586 | Park-In Alerts for Driver and Surroundings |
| --- | --- |

No Downstream References

No Associations

Description

When 
APA_status is APA_Enabled
AND
VPC_Status is other than Reset
AND
The user disables APA 
Then 
inform the driver in C-app

##### 18.12.17 --

| CodeBeamer reference: | APAVPCSR-3415878 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415590 | Park-In Maneuver terminated by Driver seated |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

When
The user press cancel option in C-App
Then
V-parking Comfort shall set VPC_canceled as true

##### 18.12.18 --

| CodeBeamer reference: | APAVPCSR-3415879 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415491 | Precise Parking Completion with KPI and Environmental Adaptation |
| --- | --- |

No Downstream References

No Associations

Description

The APA system shall perform parking maneuvers with a minimum target KPI of [APA_MIN_TARGET_KPI_PARKING_COMPLETION], while maintaining a positioning tolerance of [APA_TOLERANCE_X] cm in X direction, [APA_TOLERANCE_Y] cm in Y direction, and [ANGULAR_DEVIATION] angular deviation for increased precision.

##### 18.12.19 --

| CodeBeamer reference: | APAVPCSR-3415880 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415491 | Precise Parking Completion with KPI and Environmental Adaptation |
| --- | --- |

Downstream References (1)

| APASAD-3412216 | Sequence Diagram |
| --- | --- |

No Associations

Description

The APA system shall actively adapt to real-world constraints while ensuring a successful parking maneuver.

Note: Example would be narrow spaces, uneven surfaces

##### 18.12.20 --

| CodeBeamer reference: | APAVPCSR-3415881 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Presence of moving object/Person/Animal for more than the threshold time is true
Then
V-parking Comfort shall set VPC_Status as Abort

##### 18.12.21 --

| CodeBeamer reference: | APAVPCSR-3415882 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Remote_device_engagement is false
Then
V-parking Comfort shall set VPC_Status as Abort

##### 18.12.22 --

| CodeBeamer reference: | APAVPCSR-3415883 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Manual_override is true
Then
V-parking Comfort shall set VPC_Status as Abort

##### 18.12.23 --

| CodeBeamer reference: | APAVPCSR-3415884 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Sensor_malfunction is true
Then
V-parking Comfort shall set VPC_Status as Abort

##### 18.12.24 --

| CodeBeamer reference: | APAVPCSR-3415885 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Software_error is true
Then
V-parking Comfort shall set VPC_Status as Abort

##### 18.12.25 --

| CodeBeamer reference: | APAVPCSR-3415886 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Poor_visibility is true
Then
V-parking Comfort shall set VPC_Status as Abort

##### 18.12.26 --

| CodeBeamer reference: | APAVPCSR-3415887 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Slot_constraints is true
Then
V-parking Comfort shall set VPC_Status as Abort

##### 18.12.27 --

| CodeBeamer reference: | APAVPCSR-3415888 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Number_of_moves is greater than MAX_MOVE_TIMES
Then
V-Parking Comfort set VPC_Status as Abort

##### 18.12.28 --

| CodeBeamer reference: | APAVPCSR-3415889 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Low_battery_conditions is true
Then
V-parking Comfort shall set VPC_Status as Abort

##### 18.12.29 --

| CodeBeamer reference: | APAVPCSR-3415890 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Mechanical_or_electronic_failure is true
Then
V-parking Comfort shall set VPC_Status as Abort

##### 18.12.30 --

| CodeBeamer reference: | APAVPCSR-3415891 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Vehicle_speed is greater than MAX_AUTO_MANEUVERING_SPEED
Then
V-parking Comfort shall set VPC_Status as Abort

##### 18.12.31 --

| CodeBeamer reference: | APAVPCSR-3415892 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Improper_calibration is true
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.12.32 --

| CodeBeamer reference: | APAVPCSR-3415893 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Parking_process_time is greater than THRESHOLD_TIME
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.12.33 --

| CodeBeamer reference: | APAVPCSR-3415894 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
APA_system_failure is true
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.12.34 --

| CodeBeamer reference: | APAVPCSR-3415895 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Remote_connection_timeout is true
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.12.35 --

| CodeBeamer reference: | APAVPCSR-3415896 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
ACC_requested is true
Then
V-Parking Comfort shall set Reason_for_abort as Req_Quit_ACC (8)

##### 18.12.36 VF_PF_SYSRS_1642

| CodeBeamer reference: | APAVPCSR-3415897 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
ACC_requested is true
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.12.37 --

| CodeBeamer reference: | APAVPCSR-3415898 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
HWA_requested is true
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.12.38 --

| CodeBeamer reference: | APAVPCSR-3415899 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Deactivation_button_pressed is true
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.12.39 --

| CodeBeamer reference: | APAVPCSR-3415900 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Vehicle_blocked is true
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.12.40 --

| CodeBeamer reference: | APAVPCSR-3415901 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
PAS_failure is true
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.12.41 --

| CodeBeamer reference: | APAVPCSR-3415902 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
External_ECU_failure is true
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.12.42 --

| CodeBeamer reference: | APAVPCSR-3415903 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
RD_offline is true
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.12.43 --

| CodeBeamer reference: | APAVPCSR-3415904 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
RPA_page_off is true
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.12.44 --

| CodeBeamer reference: | APAVPCSR-3415905 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
RD_failure is true
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.12.45 --

| CodeBeamer reference: | APAVPCSR-3415906 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
RD_deactivation_request is true
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.12.46 --

| CodeBeamer reference: | APAVPCSR-3415907 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Number of moves is greater than MAX_MOVE_TIMES
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.12.47 --

| CodeBeamer reference: | APAVPCSR-3415908 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Number of interruptions is greater than MAX_INTERRUPT_TIMES
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.12.48 --

| CodeBeamer reference: | APAVPCSR-3415909 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Moving_control_timeout is true
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.12.49 --

| CodeBeamer reference: | APAVPCSR-3415910 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Excessive_pauses_detected is true
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.12.50 --

| CodeBeamer reference: | APAVPCSR-3415911 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
PAS_internal_failure is true
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.12.51 --

| CodeBeamer reference: | APAVPCSR-3415912 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
External_ECU_failure is true
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.12.52 --

| CodeBeamer reference: | APAVPCSR-3415913 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Function_conflict is true
Then
V-Parking Comfort shall set PAS_APA_ReqInfoDispTxt1 as Conflict with other functions (3)

##### 18.12.53 VF_PF_SYSRS_1643

| CodeBeamer reference: | APAVPCSR-3415914 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Function_conflict is true
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.12.54 --

| CodeBeamer reference: | APAVPCSR-3415915 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Vehicle_speed is greater than MAX_AUTO_MANEUVERING_SPEED
Then
V-Parking Comfort shall set PAS_APA_ReqInfoDispTxt1 as Vehicle speed is higher than the limit (4)

##### 18.12.55 --

| CodeBeamer reference: | APAVPCSR-3415916 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Powertrain_system_not_ready is true
Then
V-Parking Comfort shall set PAS_APA_ReqInfoDispTxt1 as The Powertrain system is not ready (5)

##### 18.12.56 VF_PF_SYSRS_1644

| CodeBeamer reference: | APAVPCSR-3415917 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Powertrain_system_not_ready is true
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.12.57 --

| CodeBeamer reference: | APAVPCSR-3415918 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Vehicle_in_towing_mode is true
Then
V-Parking Comfort shall set PAS_APA_ReqInfoDispTxt1 as Vehicle is in towing mode (6)

##### 18.12.58 VF_PF_SYSRS_1645

| CodeBeamer reference: | APAVPCSR-3415919 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Vehicle_in_towing_mode is true
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.12.59 --

| CodeBeamer reference: | APAVPCSR-3415920 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Pause_timeout is true
Then
V-Parking Comfort shall set Reason_for_abort as Req_Quit_Pause_Timeout (3)

##### 18.12.60 VF_PF_SYSRS_1646

| CodeBeamer reference: | APAVPCSR-3415921 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Pause_timeout is true
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.12.61 --

| CodeBeamer reference: | APAVPCSR-3415922 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Trajectory_deviation_detected is true
Then
V-Parking Comfort shall set Reason_for_abort as Req_Quit_Trajectory (4)

##### 18.12.62 VF_PF_SYSRS_1647

| CodeBeamer reference: | APAVPCSR-3415923 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Trajectory_deviation_detected is true
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.12.63 --

| CodeBeamer reference: | APAVPCSR-3415924 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
No_valid_trajectory_found is true
Then
V-Parking Comfort shall set Reason_for_abort as Req_Quit_Trajectory (4)

##### 18.12.64 VF_PF_SYSRS_1648

| CodeBeamer reference: | APAVPCSR-3415925 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
No_valid_trajectory_found is true
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.12.65 --

| CodeBeamer reference: | APAVPCSR-3415926 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Space_limit_exceeded is true
Then
V-Parking Comfort shall set Reason_for_abort as Req_Quit_Space_Limit (5)

##### 18.12.66 VF_PF_SYSRS_1649

| CodeBeamer reference: | APAVPCSR-3415927 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Space_limit_exceeded is true
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.12.67 --

| CodeBeamer reference: | APAVPCSR-3415928 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Slope_exceeds_limitation is true
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.12.68 --

| CodeBeamer reference: | APAVPCSR-3415929 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Unsafe_behavior_detected is true
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.12.69 --

| CodeBeamer reference: | APAVPCSR-3415930 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Remote_connection_timeout is true
Then
V-Parking Comfort shall set Reason_for_abort as Req_Quit_Remote_Connection_Timeout (1E)

##### 18.12.70 --

| CodeBeamer reference: | APAVPCSR-3415931 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
RD_low_battery is true
Then
V-Parking Comfort shall set Reason_for_abort as Req_Quit_RD_Low_Batt (19)

##### 18.12.71 VF_PF_SYSRS_1650

| CodeBeamer reference: | APAVPCSR-3415932 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
RD_low_battery is true
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.12.72 --

| CodeBeamer reference: | APAVPCSR-3415933 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
BCM_STAT_BonnetAjar is Open
Then
V-Parking Comfort shall set Reason_for_abort as Req_Quit_Engine_Front_Door_Open (15)

##### 18.12.73 VF_PF_SYSRS_1651

| CodeBeamer reference: | APAVPCSR-3415934 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
BCM_STAT_BonnetAjar is Open
Then
V-Parking Comfort shall set VPC_Status as Abort

##### 18.12.74 --

| CodeBeamer reference: | APAVPCSR-3415935 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Steering_wheel_intervention is true
Then
V-Parking Comfort shall set Reason_for_abort as Req_Quit_Steering_Wheel_Intervention (11)

##### 18.12.75 VF_PF_SYSRS_1652

| CodeBeamer reference: | APAVPCSR-3415936 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering /moving or suspend or finished
AND
Steering_wheel_intervention is true
Then
V-Parking Comfort shall set VPC_Status as Abort

#### 18.13 Reset:

| CodeBeamer reference: | APAVPCSR-3415730 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.13.1 Description

| CodeBeamer reference: | APAVPCSR-3415937 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

VPC will enter Reset state and apply default settings if user preferences are not selected or if a reset fault occurs, ensuring safe and consistent operation.

##### 18.13.2 Input Signals

| CodeBeamer reference: | APAVPCSR-3415938 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.13.3 Output Signals

| CodeBeamer reference: | APAVPCSR-3415939 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.13.4 Internal Signals

| CodeBeamer reference: | APAVPCSR-3415940 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.13.5 --

| CodeBeamer reference: | APAVPCSR-3415941 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

No Upstream References

No Downstream References

No Associations

Description

When
VPC_Status is Reset
Then
V-Parking Comfort shall reset all user requests to default values.

#### 18.14 Emergency Stop/Take Back Control:

| CodeBeamer reference: | APAVPCSR-3415731 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.14.1 Description

| CodeBeamer reference: | APAVPCSR-3415942 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

If an obstacle persists beyond the allowed time or unexpected conditions arise during park-out, the system shall abort the maneuver and allow the driver to take back control for safety.

##### 18.14.2 Input Signals

| CodeBeamer reference: | APAVPCSR-3415943 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.14.3 Output Signals

| CodeBeamer reference: | APAVPCSR-3415944 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.14.4 Internal Signals

| CodeBeamer reference: | APAVPCSR-3415945 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.14.5 --

| CodeBeamer reference: | APAVPCSR-3415946 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

When
VPC_Status is path maneuvering/moving
AND
Obstcl_Identified is true for more than MAX_OBSTACLE_WAIT_TIME
Then
V-Parking Comfort shall set VPC_Status as abort and inform driver

##### 18.14.6 --

| CodeBeamer reference: | APAVPCSR-3415947 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415598 | Low-speed navigation: Initiation via MHU |
| --- | --- |
| APAVPC-3415597 | Park-In Maneuver terminated by Remote Driver |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When
VPC_Parking_Active is active
AND
Park_type is Park out
Then
V-Parking Comfort shall plan the path with the recorded data and real time input

##### 18.14.7 --

| CodeBeamer reference: | APAVPCSR-3415948 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415597 | Park-In Maneuver terminated by Remote Driver |
| --- | --- |

Downstream References (1)

| APASAD-3412216 | Sequence Diagram |
| --- | --- |

No Associations

Description

When
VPC_Parking_Active is active
AND
Park_type is Park out
Then
V-Parking Comfort shall display the planned trajectory in C-App (VPC_C-App_Park-out_Trajectory) and MHU (VPC_MHU_Park-out_Trajectory) and arrival time

##### 18.14.8 --

| CodeBeamer reference: | APAVPCSR-3415949 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415597 | Park-In Maneuver terminated by Remote Driver |
| --- | --- |

Downstream References (1)

| APASAD-3412216 | Sequence Diagram |
| --- | --- |

No Associations

Description

When
VPC_Parking_Active is active
AND
Park_type is Park out
Then
V-Parking Comfort shall display the progress bar with respect to current position in C-App

##### 18.14.9 --

| CodeBeamer reference: | APAVPCSR-3415950 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415593 | V_Comfort Park Out Suggestion |
| --- | --- |

Downstream References (1)

| APASAD-3412216 | Sequence Diagram |
| --- | --- |

No Associations

Description

The APA system shall permit automatic Park out maneuvers only if the vehicle was previously parked using VPC Park in

#### 18.15 Path-Planning and selection:

| CodeBeamer reference: | APAVPCSR-3415732 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.15.1 Description

| CodeBeamer reference: | APAVPCSR-3415951 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

The system shall compute and display feasible trajectories based on user input, parking type, and real-time conditions, ensuring only valid paths are planned and shown when all preconditions are met.

##### 18.15.2 Input Signals

| CodeBeamer reference: | APAVPCSR-3415952 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

MHU_Selected_ Slot_ID
VCU_Veh_Spd
VCU_Gear_State
C-App_Selected_ Slot_ID
MHU_Selected_Trajectory
C-App_Selected_Park-out_Dir
Keyfob_Auto_Park-out

##### 18.15.3 Output Signals

| CodeBeamer reference: | APAVPCSR-3415953 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

VPC_Parking_Active
VPC_MHU_Nose-out_feasible
VPC_MHU_Nose-in_feasible

##### 18.15.4 Internal Signals

| CodeBeamer reference: | APAVPCSR-3415954 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

Trace_Path 
Nose-in_feasible
Nose-out_feasible

##### 18.15.5 --

| CodeBeamer reference: | APAVPCSR-3415955 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415580 | Trajectory Calculation Based on Driver’s Slot and Direction Selection |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When
VPC_Parking_Active is True
AND
Park_type is Park in
AND
VPC_Status is Path-planning and selection
AND
MHU_Selected_ Slot_ID value is received from MHU
AND
VCU_Veh_Spd is 0
AND
VCU_Gear_State is Park
Then
V-Parking Comfort shall set Target_Coordinate as MHU_Selected_ Slot_ID.Slot_Coordinate.

##### 18.15.6 VF_PF_SYSRS_1653

| CodeBeamer reference: | APAVPCSR-3415956 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

Upstream References (1)

| APAVPC-3415580 | Trajectory Calculation Based on Driver’s Slot and Direction Selection |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Parking_Active is True
AND
Park_type is Park in
AND
VPC_Status is Path-planning and selection
AND
C-App_Selected_ Slot_ID value is received from C-App
AND
VCU_Veh_Spd is 0
AND
VCU_Gear_State is Park
Then
V-Parking Comfort shall set Target_Coordinate as C-App_Selected_ Slot_ID.Slot_Coordinate.

##### 18.15.7 VF_PF_SYSRS_1654

| CodeBeamer reference: | APAVPCSR-3415957 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

Upstream References (2)

| APAVPC-3415583 | Display Park-In Trajectory and progress |
| --- | --- |
| APAVPC-3415580 | Trajectory Calculation Based on Driver’s Slot and Direction Selection |

No Downstream References

No Associations

Description

When
VPC_Parking_Active is True
AND
Park_type is Park in
AND
MHU_Selected_ Slot_ID value is received from MHU
OR
C-App_Selected_ Slot_ID value is received from C-App
AND
VPC_Status is Path-planning and selection
Then
V-Parking Comfort shall invoke Path-Planning Module for Nose-out with current vehicle position(TCU_Current_Veh_Posn) and Target_Coordinate to receive Trajectory_Info

##### 18.15.8 VF_PF_SYSRS_1655

| CodeBeamer reference: | APAVPCSR-3415958 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415583 | Display Park-In Trajectory and progress |
| --- | --- |
| APAVPC-3415580 | Trajectory Calculation Based on Driver’s Slot and Direction Selection |

No Downstream References

No Associations

Description

When
VPC_Parking_Active is True
AND
Park_type is Park in
AND
VPC_Status is Path Planning and Selection
AND
Trajectory_Info.direction = Nose-out
AND
Trajectory_Info.feasibility_flag is True
Then
V-Parking Comfort shall set Nose-out_feasible to True

##### 18.15.9 VF_PF_SYSRS_1656

| CodeBeamer reference: | APAVPCSR-3415959 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415583 | Display Park-In Trajectory and progress |
| --- | --- |
| APAVPC-3415580 | Trajectory Calculation Based on Driver’s Slot and Direction Selection |

No Downstream References

No Associations

Description

When
VPC_Parking_Active is True
AND
Park_type is Park in
AND
VPC_Status is Path Planning and Selection
AND
MHU_Selected_ Slot_ID value is received from MHU
OR
C-App_Selected_ Slot_ID value is received from C-App
Then
V-Parking Comfort shall invoke Path-Planning Module for Nose-in with current vehicle position(TCU_Current_Veh_Posn) and Target_Coordinate to receive Trajectory_Info 

Note: Path Planning Module will calculate the possible trajectories and arrival time for Nose-out and Nose-in.

##### 18.15.10 VF_PF_SYSRS_1657

| CodeBeamer reference: | APAVPCSR-3415960 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415583 | Display Park-In Trajectory and progress |
| --- | --- |
| APAVPC-3415580 | Trajectory Calculation Based on Driver’s Slot and Direction Selection |

No Downstream References

No Associations

Description

When
VPC_Parking_Active is True
AND
Park_type is Park in
AND
VPC_Status is Path Planning and Selection
AND
Trajectory_Info.direction = Nose-in
AND
Trajectory_Info.feasibility_flag is True
Then
V-Parking Comfort shall set Nose-in_feasible to True

##### 18.15.11 VF_PF_SYSRS_1658

| CodeBeamer reference: | APAVPCSR-3415961 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415580 | Trajectory Calculation Based on Driver’s Slot and Direction Selection |
| --- | --- |
| APAVPC-3415583 | Display Park-In Trajectory and progress |

No Downstream References

No Associations

Description

Path Planning and Selection to Slot Selection
When
Park_Type is Park-in
AND
VPC_Status is Path Planning and Selection
AND
Nose-in_feasible is False (Nose in Parking is not feasible)
AND 
Nose-out_feasible is False (Nose out Parking is not feasible)
Then
V-Parking Selec shall send VPC_Parking_NotFeasibleTxt to inform the driver that no feasible path exists to reach the selected slot and to select another slot.

##### 18.15.12 --

| CodeBeamer reference: | APAVPCSR-3415962 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415580 | Trajectory Calculation Based on Driver’s Slot and Direction Selection |
| --- | --- |
| APAVPC-3415583 | Display Park-In Trajectory and progress |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When
VPC_Parking_Active is True
AND
Park_type is Park in
AND
MHU_Selected_ Slot_ID is received from MHU
AND
VPC_Status is Path-planning and selection
AND
Nose-in_feasible is True (Nose in Parking is feasible )
AND
Nose-out_feasible is True (Nose out Parking is feasible) 
Then
V-Parking Comfort shall set VPC_MHU_Nose-out_feasible as True and VPC_MHU_Nose-in_feasible as True
e

##### 18.15.13 VF_PF_SYSRS_1659

| CodeBeamer reference: | APAVPCSR-3415963 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

When
VPC_Parking_Active is True
AND
Park_type is Park in
AND
MHU_Selected_ Slot_ID is received from MHU
AND
VPC_Status is Path-planning and selection
AND
Nose-in_feasible is True (Nose in Parking is feasible )
AND
Nose-out_feasible is True (Nose out Parking is feasible) 
Then
The V-Parking Comfort shall set (Trajectory_Info.Details to VPC_MHU_Nose-out_Trajectory and Trajectory_Info.CompltnTime to VPC_MHU_Nose-out_CompltnTime for the selected slot(MHU_Selected_ Slot_ID) and Trajectory_Info.Direction = Nose-out) for MHU display.

##### 18.15.14 VF_PF_SYSRS_1660

| CodeBeamer reference: | APAVPCSR-3415964 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

When
VPC_Parking_Active is True
AND
Park_type is Park in
AND
C-App_Selected_ Slot_ID is received from C-App
AND
VPC_Status is Path-planning and selection
AND
Nose-in_feasible is True (Nose in Parking is feasible )
AND
Nose-out_feasible is True (Nose out Parking is feasible) 
Then
V-Parking Comfort shall set VPC_C-App_Nose-out_feasible as True and VPC_C-App_Nose-in_feasible as True

##### 18.15.15 VF_PF_SYSRS_1661

| CodeBeamer reference: | APAVPCSR-3415965 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

When
VPC_Parking_Active is True
AND
Park_type is Park in
AND
C-App_Selected_ Slot_ID is received from C-App
AND
VPC_Status is Path-planning and selection
AND
Nose-in_feasible is True (Nose in Parking is feasible )
AND
Nose-out_feasible is True (Nose out Parking is feasible) 
Then
The V-Parking Comfort shall set (Trajectory_Info.Details to VPC_C-App_Nose-out_Trajectory and Trajectory_Info.CompltnTime to VPC_C-App_Nose-out_CompltnTime for the selected slot(C-App_Selected_ Slot_ID) and Trajectory_Info.Direction = Nose-out) for C-App display.

##### 18.15.16 --

| CodeBeamer reference: | APAVPCSR-3415966 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415580 | Trajectory Calculation Based on Driver’s Slot and Direction Selection |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When
VPC_Parking_Active is True
AND
Park_type is Park in
AND
MHU_Selected_ Slot_ID value is received from MHU
AND
VPC_Status is Path Planning and Selection
AND
Nose-out_feasible is True
AND
Nose-in_feasible is True 
AND
MHU_Nose-in_Req is True 
Then
The V-Parking Comfort shall set (Trajectory_Info.Details to VPC_MHU_Nose-in_Trajectory and Trajectory_Info.CompltnTime to VPC_MHU_Nose-out_CompltnTime for the selected slot(MHU_Selected_ Slot_ID) and Trajectory_Info.Direction = Nose-in) for MHU display.

##### 18.15.17 VF_PF_SYSRS_1662

| CodeBeamer reference: | APAVPCSR-3415967 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

When
VPC_Parking_Active is True
AND
Park_type is Park in
AND
C-App_Selected_ Slot_ID value is received from C-App
AND
VPC_Status is Path Planning and Selection
AND
Nose-out_feasible is True
AND
Nose-in_feasible is True 
AND
C-App_Nose-in_Req is True 
Then
The V-Parking Comfort shall set (Trajectory_Info.Details to VPC_C-App_Nose-in_Trajectory and Trajectory_Info.CompltnTime to VPC_C-App_Nose-out_CompltnTime for the selected slot(C-App_Selected_ Slot_ID) and Trajectory_Info.Direction = Nose-in) for C-App display.

##### 18.15.18 --

| CodeBeamer reference: | APAVPCSR-3415968 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415580 | Trajectory Calculation Based on Driver’s Slot and Direction Selection |
| --- | --- |
| APAVPC-3415583 | Display Park-In Trajectory and progress |

Downstream References (1)

| APASAD-3412216 | Sequence Diagram |
| --- | --- |

No Associations

Description

When
VPC_Parking_Active is True
AND
Park_type is Park in
AND
MHU_Selected_ Slot_ID value is received from MHU
AND
VPC_Status is Path-planning and selection
AND
Nose-out_feasible is True (Nose out Parking is feasible)
AND
Nose-in_feasible is False (Nose in Parking is not feasible)
Then
V-Parking Comfort shall set VPC_MHU_Nose-out_feasible as True and VPC_MHU_Nose-in_feasible as False

##### 18.15.19 VF_PF_SYSRS_1663

| CodeBeamer reference: | APAVPCSR-3415969 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

When
VPC_Parking_Active is True
AND
Park_type is Park in
AND 
MHU_Selected_ Slot_ID value is received from MHU
AND
VPC_Status is Path Planning and Selection
AND
Nose-out_feasible is True
AND
Nose-in_feasible is False 
Then
V-Parking Comfort shall set (Trajectory_Info.Details to VPC_MHU_Nose-out_Trajectory and Trajectory_Info.CompltnTime to VPC_MHU_Nose-out_CompltnTime for the selected slot(MHU_Selected_ Slot_ID) and Trajectory_Info.Direction = Nose-out) for MHU display

##### 18.15.20 VF_PF_SYSRS_1664

| CodeBeamer reference: | APAVPCSR-3415970 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

When
VPC_Parking_Active is True
AND
Park_type is Park in
AND
C-App_Selected_ Slot_ID value is received from MHU
AND
VPC_Status is Path-planning and selection
AND
Nose-out_feasible is True (Nose out Parking is feasible)
AND
Nose-in_feasible is False (Nose in Parking is not feasible)
Then
V-Parking Comfort shall set VPC_C-App_Nose-out_feasible as True and VPC_C-APP_Nose-in_feasible as False

##### 18.15.21 VF_PF_SYSRS_1665

| CodeBeamer reference: | APAVPCSR-3415971 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

When
VPC_Parking_Active is True
AND
Park_type is Park in
AND 
C-App_Selected_ Slot_ID value is received from C-App
AND
VPC_Status is Path Planning and Selection
AND
Nose-out_feasible is True
AND
Nose-in_feasible is False 
Then
V-Parking Comfort shall set (Trajectory_Info.Details to VPC_C-App_Nose-out_Trajectory and Trajectory_Info.CompltnTime to VPC_C-App_Nose-out_CompltnTime for the selected slot(MHU_Selected_ Slot_ID) and Trajectory_Info.Direction = Nose-out) for C-App display

##### 18.15.22 --

| CodeBeamer reference: | APAVPCSR-3415972 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415580 | Trajectory Calculation Based on Driver’s Slot and Direction Selection |
| --- | --- |

Downstream References (1)

| APASAD-3412216 | Sequence Diagram |
| --- | --- |

No Associations

Description

When
VPC_Parking_Active is True
AND
Park_type is Park in
AND
MHU_Selected_ Slot_ID value is received from MHU
AND
VPC_Status is Path Planning and Selection
AND
Nose-out_feasible is False (Nose out Parking is not feasible)
AND
Nose-in_feasible is True (Nose in Parking is feasible)
Then
V-Parking Comfort shall set VPC_MHU_Nose-out_feasible as False and VPC_MHU_Nose-in_feasible as True

##### 18.15.23 VF_PF_SYSRS_1666

| CodeBeamer reference: | APAVPCSR-3415973 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (3)

| APAVPC-3415581 | Persistent Obstacle: Abort maneuver by System |
| --- | --- |
| APAVPC-3415582 | Prompt Driver for Slot reselection on Trajectory failure |
| APAVPC-3415580 | Trajectory Calculation Based on Driver’s Slot and Direction Selection |

No Downstream References

No Associations

Description

When
VPC_Parking_Active is True
AND
Park_type is Park in
AND
MHU_Selected_ Slot_ID value is received from MHU
AND
VPC_Status is Path Planning and Selection
AND
Nose-out_feasible is False
AND
Nose-in_feasible is True
Then
The V-Parking Comfort shall set (Trajectory_Info.Details to VPC_MHU_Nose-in_Trajectory and Trajectory_Info.CompltnTime to VPC_MHU_Nose-in_CompltnTime for the selected slot(MHU_Selected_ Slot_ID) and Trajectory_Info.Direction = Nose-in) for MHU display.

##### 18.15.24 VF_PF_SYSRS_1667

| CodeBeamer reference: | APAVPCSR-3415974 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

When
VPC_Parking_Active is True
AND
Park_type is Park in
AND
C-App_Selected_ Slot_ID value is received from C-App
AND
VPC_Status is Path Planning and Selection
AND
Nose-out_feasible is False (Nose out Parking is not feasible)
AND
Nose-in_feasible is True (Nose in Parking is feasible)
Then
V-Parking Comfort shall set VPC_C-App_Nose-out_feasible as False and VPC_C-App_Nose-in_feasible as True

##### 18.15.25 VF_PF_SYSRS_1668

| CodeBeamer reference: | APAVPCSR-3415975 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

When
VPC_Parking_Active is True
AND
Park_type is Park in
AND
C-App_Selected_ Slot_ID value is received from C-App
AND
VPC_Status is Path Planning and Selection
AND
Nose-out_feasible is False
AND
Nose-in_feasible is True
Then
The V-Parking Comfort shall set (Trajectory_Info.Details to VPC_C-App_Nose-in_Trajectory and Trajectory_Info.CompltnTime to VPC_C-App_Nose-in_CompltnTime for the selected slot(C-App_Selected_ Slot_ID) and Trajectory_Info.Direction = Nose-in) for C-App display.

##### 18.15.26 VF_PF_SYSRS_1669

| CodeBeamer reference: | APAVPCSR-3415976 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (3)

| APAVPC-3415581 | Persistent Obstacle: Abort maneuver by System |
| --- | --- |
| APAVPC-3415584 | Park-In Maneuver Initiation: Control mode change |
| APAVPC-3415580 | Trajectory Calculation Based on Driver’s Slot and Direction Selection |

No Downstream References

No Associations

Description

When
Park_type is Park in
AND
VPC_Status is Path-planning and selection
AND
MHU_Selected_Trajectory is received from MHU 
Then
The V-Parking shall set Trace_Path as MHU_Selected_Trajectory

##### 18.15.27 VF_PF_SYSRS_1670

| CodeBeamer reference: | APAVPCSR-3415977 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (3)

| APAVPC-3415584 | Park-In Maneuver Initiation: Control mode change |
| --- | --- |
| APAVPC-3415580 | Trajectory Calculation Based on Driver’s Slot and Direction Selection |
| APAVPC-3415581 | Persistent Obstacle: Abort maneuver by System |

No Downstream References

No Associations

Description

When
Park_type is Park in
AND
VPC_Status is Path-planning and selection
AND
C-App_Selected_Trajectory is received from C-App
Then
The V-Parking shall set Trace_Path as C-App_Selected_Trajectory

##### 18.15.28 VF_PF_SYSRS_1671

| CodeBeamer reference: | APAVPCSR-3415978 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415583 | Display Park-In Trajectory and progress |
| --- | --- |

No Downstream References

No Associations

Description

When
Park_type is Park in
AND
VPC_Status is Path Planning and Selection
AND
MHU_Selected_Trajectory is received from MHU 
Then
The V-Parking shall set VPC_ParkIn_Trajectory_Selected as True

##### 18.15.29 VF_PF_SYSRS_1672

| CodeBeamer reference: | APAVPCSR-3415979 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415583 | Display Park-In Trajectory and progress |
| --- | --- |

No Downstream References

No Associations

Description

When
Park_type is Park in
AND
VPC_Status is Path Planning and Selection
AND
C-App_Selected_Trajectory is received from C-App
Then
The V-Parking shall set VPC_ParkIn_Trajectory_Selected as True

##### 18.15.30 --

| CodeBeamer reference: | APAVPCSR-3415980 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415597 | Park-In Maneuver terminated by Remote Driver |
| --- | --- |
| APAVPC-3415598 | Low-speed navigation: Initiation via MHU |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When
VPC_Parking_Active is True
AND
C-App_Selected_Park-out_Dir is received from C-App
AND
VPC_Status is Path-planning and selection
AND
C-App_Selected_Park-out_Dir is received from C-App
Then
V-Parking Comfort shall invoke Path-Planning Module for park-out with current vehicle position(TCU_Current_Veh_Posn) and C-App_Selected_Park-out_Dir to generate Trajectory_Info 

Note: Path Planning Module will calculate the possible trajectory and arrival time for bringing the vehicle out of the parking lot and place along the driving side of the road . 
Trajectory_Info (Trajectory_Info.Details, Trajectory_Info.direction, Trajectory_Info.Feasibility_flag, Trajectory_Info.CompltnTime)

##### 18.15.31 VF_PF_SYSRS_1673

| CodeBeamer reference: | APAVPCSR-3415981 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (3)

| APAVPC-3415597 | Park-In Maneuver terminated by Remote Driver |
| --- | --- |
| APAVPC-3415598 | Low-speed navigation: Initiation via MHU |
| APAVPC-3415583 | Display Park-In Trajectory and progress |

No Downstream References

No Associations

Description

When
VPC_Parking_Active is True
AND
C-App_Park_Type_Req is Park-out
AND
VPC_Status is Path Planning and Selection
AND
C-App_Selected_Park-out_Dir is received from C-App
Then
The V-Parking Comfort shall set (Trajectory_Info.Details to VPC_ParkOut_Trajectory and Trajectory_Info.CompltnTime to VPC_ParkOut_CompltnTime for the selected slot(C-App_Selected_ Slot_ID) and Trajectory_Info.Direction = C-App_Selected_Park-out_Dir) for C-App display.

##### 18.15.32 --

| CodeBeamer reference: | APAVPCSR-3415982 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415597 | Park-In Maneuver terminated by Remote Driver |
| --- | --- |

Downstream References (1)

| APASAD-3412216 | Sequence Diagram |
| --- | --- |

No Associations

Description

When
Keyfob_Auto_Park-out is True
AND
VPC_Parking_Active is True
AND
Park_type is Park out
AND
VPC_Status is Path-planning and selection
Then
V-Parking Comfort shall set the Trace_Path as default trajectory_0

Note: If default_trajectory_0 is selected, V-Parking Comfort shall decide on an ideal park-out direction (which ever is lower in complexity and completion time) and generate the trajectory.

#### 18.16 Statistics Reporting:

| CodeBeamer reference: | APAVPCSR-3415733 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.16.1 Descrption

| CodeBeamer reference: | APAVPCSR-3415983 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

The APA system shall securely collect, transmit, and store maneuver data—including difficulty, success rate, and duration—for real-time insights and long-term analysis.

##### 18.16.2 --

| CodeBeamer reference: | APAVPCSR-3415984 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415616 | Deactivation: Driver Cancel |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

The system shall integrate with telematics systems to capture detailed statistics including maneuver difficulty (categorized by predefined levels), success rate (percentage of successful maneuvers), and time taken (measured in seconds).

##### 18.16.3 --

| CodeBeamer reference: | APAVPCSR-3415985 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415616 | Deactivation: Driver Cancel |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

Data shall be encrypted and securely transmitted to VinFast and other stakeholders using industry-standard protocols (e.g., TLS/SSL).

##### 18.16.4 --

| CodeBeamer reference: | APAVPCSR-3415986 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415616 | Deactivation: Driver Cancel |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

The system shall provide real-time reporting capabilities and store historical data for a minimum of MAX_VALIDITY_PERIOD for trend analysis.

#### 18.17 Request Validation:

| CodeBeamer reference: | APAVPCSR-3415734 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.17.1 Descrption

| CodeBeamer reference: | APAVPCSR-3415987 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

The system shall ensure parking requests are authenticated, protected against malicious actions, and securely logged in the cloud without storing personal user data.

##### 18.17.2 --

| CodeBeamer reference: | APAVPCSR-3415988 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415619 | Driver Request Validation and Fault/Malicious Trigger Prevention |
| --- | --- |

No Downstream References

No Associations

Description

The APA system shall implement multi-factor authentication (MFA) to ensure all parking requests are driver-initiated.

##### 18.17.3 --

| CodeBeamer reference: | APAVPCSR-3415989 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415619 | Driver Request Validation and Fault/Malicious Trigger Prevention |
| --- | --- |

No Downstream References

No Associations

Description

The APA system shall include diagnostic checks to detect and mitigate MHU faults and employ intrusion detection systems (IDS) to identify and block malicious third-party attacks.

##### 18.17.4 --

| CodeBeamer reference: | APAVPCSR-3415990 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415619 | Driver Request Validation and Fault/Malicious Trigger Prevention |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

Logs of all parking requests and validation results shall be maintained for audit purposes.

#### 18.18 Firewall and Security:

| CodeBeamer reference: | APAVPCSR-3415735 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.18.1 Descrption

| CodeBeamer reference: | APAVPCSR-3415991 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

The system shall actively prevent unauthorized commands and cyber-attacks, immediately deactivate functions upon detection, and alert the user to ensure secure operation.

##### 18.18.2 --

| CodeBeamer reference: | APAVPCSR-3415992 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415620 | Firewall Protection for Secure Driver Command Handling » |
| --- | --- |

No Downstream References

No Associations

Description

The APA system shall incorporate a firewall with deep packet inspection (DPI) capabilities to block unauthorized commands from external sources via Bluetooth or Wi-Fi.

##### 18.18.3 --

| CodeBeamer reference: | APAVPCSR-3415993 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415620 | Firewall Protection for Secure Driver Command Handling » |
| --- | --- |

No Downstream References

No Associations

Description

The Automated Parking Assistance (APA) system shall automatically deactivate and alert the driver in the event of a detected cyber attack

##### 18.18.4 --

| CodeBeamer reference: | APAVPCSR-3415994 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415620 | Firewall Protection for Secure Driver Command Handling » |
| --- | --- |

No Downstream References

No Associations

Description

Regular security audits and penetration testing shall be conducted to ensure the firewall's effectiveness.

#### 18.19 OTA Updates:

| CodeBeamer reference: | APAVPCSR-3415736 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.19.1 Descrption

| CodeBeamer reference: | APAVPCSR-3415995 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

The system shall support secure OTA updates for features and security patches, initiated only with user confirmation and when all preconditions are met, without affecting core functionalities.

##### 18.19.2 --

| CodeBeamer reference: | APAVPCSR-3415996 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415621 | Support OTA Updates |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

The APA system shall support over-the-air (OTA) updates for patching vulnerabilities, enhancing security features, and performing software upgrades.

##### 18.19.3 --

| CodeBeamer reference: | APAVPCSR-3415997 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415621 | Support OTA Updates |
| --- | --- |

No Downstream References

No Associations

Description

The APA system shall ensure minimal disruption to vehicle operations during updates by scheduling them during non-peak hours or when the vehicle is not in use.

##### 18.19.4 --

| CodeBeamer reference: | APAVPCSR-3415998 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415621 | Support OTA Updates |
| --- | --- |

No Downstream References

No Associations

Description

APA system shall notify users of available updates and provide the option to defer installation if desired.

#### 18.20 Connectivity:

| CodeBeamer reference: | APAVPCSR-3415737 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.20.1 Descrption

| CodeBeamer reference: | APAVPCSR-3415999 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

VPC shall ensure seamless and resilient communication with remote devices by maintaining stable Wi-Fi and Bluetooth links, enabling rapid reconnection within MAX_RECNT_TIME (5) seconds and leveraging redundant channels for uninterrupted operation.

##### 18.20.2 --

| CodeBeamer reference: | APAVPCSR-3416000 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415617 | Remote Interface Connectivity |
| --- | --- |

No Downstream References

No Associations

Description

The APA system shall maintain continuous connectivity with remote devices using Wi-Fi (802.11ac or higher) and Bluetooth (version 5.0 or higher) technologies.

##### 18.20.3 --

| CodeBeamer reference: | APAVPCSR-3416001 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415617 | Remote Interface Connectivity |
| --- | --- |

No Downstream References

No Associations

Description

The APA system shall prioritize quick reconnection in case of connectivity loss, with a maximum reconnection time of MAX_RECONNCT_TIME (5) seconds.

##### 18.20.4 --

| CodeBeamer reference: | APAVPCSR-3416002 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415617 | Remote Interface Connectivity |
| --- | --- |

No Downstream References

No Associations

Description

Redundant communication channels shall be implemented to ensure reliability.

#### 18.21 Communication Control:

| CodeBeamer reference: | APAVPCSR-3415738 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.21.1 Descrption

| CodeBeamer reference: | APAVPCSR-3416003 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

To ensure distraction-free parking, the system shall temporarily manage incoming calls and alerts, inform the driver through clear notifications, and automatically restore communication once the maneuver is complete.

##### 18.21.2 --

| CodeBeamer reference: | APAVPCSR-3416004 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415618 | Low-speed navigation: Initiation via C-App |
| --- | --- |

No Downstream References

No Associations

Description

During automatic parking, V-Parking Comfort shall have the capability to temporarily control incoming calls and communications with remote devices.

##### 18.21.3 --

| CodeBeamer reference: | APAVPCSR-3416005 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415618 | Low-speed navigation: Initiation via C-App |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

The V-Parking Comfort shall provide visual and auditory notifications to the driver about the temporary control of communications.

##### 18.21.4 --

| CodeBeamer reference: | APAVPCSR-3416006 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415618 | Low-speed navigation: Initiation via C-App |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

Normal communication functions shall resume automatically once parking is complete, with a confirmation notification to the driver. [!ebe5aa6597ba8a5eab2f64a4807bdc81.png!] [!1244d91748f66b45030c73942b7217d4.png!] [!fe382ded91ea2ac2342635655915dd93.png!]

#### 18.22 Occupancy Monitoring:

| CodeBeamer reference: | APAVPCSR-3415739 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

null [!ebe5aa6597ba8a5eab2f64a4807bdc81.png!] [!1244d91748f66b45030c73942b7217d4.png!] [!fe382ded91ea2ac2342635655915dd93.png!]

##### 18.22.1 Descrption

| CodeBeamer reference: | APAVPCSR-3416007 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

The system shall use anonymized cabin data to adapt parking maneuvers in real time, ensuring safety and efficiency based on vehicle occupancy during automated operations.

##### 18.22.2 --

| CodeBeamer reference: | APAVPCSR-3416008 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415622 | Low-speed navigation: Initiation |
| --- | --- |

No Downstream References

No Associations

Description

The V-Parking Comfort shall adjust parking maneuvers based on occupancy data to ensure safety, such as avoiding sudden stops or sharp turns if passengers are detected.

##### 18.22.3 --

| CodeBeamer reference: | APAVPCSR-3416009 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415622 | Low-speed navigation: Initiation |
| --- | --- |
| APAVPC-3415589 | Park-In Completion Activities: Remote driver |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

V-Parking Comfort shall anonymize occupancy data and use it solely to enhance parking safety and efficiency.

##### 18.22.4 --

| CodeBeamer reference: | APAVPCSR-3416010 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415622 | Low-speed navigation: Initiation |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

The V-Parking Comfort shall utilize the Cabin Monitoring System to monitor vehicle occupancy during automatic parking.

##### 18.22.5 --

| CodeBeamer reference: | APAVPCSR-3416011 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

TBD (This section will be enhanced further in the reverse engineering phase)

#### 18.23 Safety Mechanism related to detection, indication and control of faults in the system

| CodeBeamer reference: | APAVPCSR-3415740 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.23.1 Descrption

| CodeBeamer reference: | APAVPCSR-3416012 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

Upstream References (1)

| APAVPC-3415616 | Deactivation: Driver Cancel |
| --- | --- |

No Downstream References

No Associations

Description

--

###### 18.23.1.1 --

| CodeBeamer reference: | APAVPCSR-3416297 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

TBD, the section will be updated once the information is available

#### 18.24 Safety Mechanism related to detection, indication and control of faults in external systems interacting with the system

| CodeBeamer reference: | APAVPCSR-3415741 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.24.1 Descrption

| CodeBeamer reference: | APAVPCSR-3416013 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

Upstream References (1)

| APAVPC-3415619 | Driver Request Validation and Fault/Malicious Trigger Prevention |
| --- | --- |

No Downstream References

No Associations

Description

--

#### 18.25 Safety Mechanism for the measures that enable system to achieve / maintain safe state

| CodeBeamer reference: | APAVPCSR-3415742 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.25.1 Descrption

| CodeBeamer reference: | APAVPCSR-3416014 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

Upstream References (1)

| APAVPC-3415620 | Firewall Protection for Secure Driver Command Handling » |
| --- | --- |

No Downstream References

No Associations

Description

--

#### 18.26 Safety Mechanism for measures to detail and implement the warning and degradation concept

| CodeBeamer reference: | APAVPCSR-3415743 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.26.1 Descrption

| CodeBeamer reference: | APAVPCSR-3416015 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

Upstream References (1)

| APAVPC-3415621 | Support OTA Updates |
| --- | --- |

No Downstream References

No Associations

Description

--

##### 18.26.2 Safety Mechanism for avoiding Latent fault

| CodeBeamer reference: | APAVPCSR-3416016 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

###### 18.26.2.1 Descrption

| CodeBeamer reference: | APAVPCSR-3416298 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

Upstream References (1)

| APAVPC-3415617 | Remote Interface Connectivity |
| --- | --- |

No Downstream References

No Associations

Description

--

#### 18.27 Requirements for production, operation, service and decommissioning

| CodeBeamer reference: | APAVPCSR-3415744 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 18.27.1 Descrption

| CodeBeamer reference: | APAVPCSR-3416017 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

Upstream References (1)

| APAVPC-3415618 | Low-speed navigation: Initiation via C-App |
| --- | --- |

No Downstream References

No Associations

Description

--

##### 18.27.2 --

| CodeBeamer reference: | APAVPCSR-3416018 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

should be done from ADAS ECU perspective not feature specific

### 19 Non Functional Requirements

| CodeBeamer reference: | APAVPCSR-3415653 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 19.1 Performance Requirements

| CodeBeamer reference: | APAVPCSR-3415745 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 19.1.1 --

| CodeBeamer reference: | APAVPCSR-3416019 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

To be updated once System KPI Pre-sys activity is completed

##### 19.1.2 VF_PF_SYSRS_1675

| CodeBeamer reference: | APAVPCSR-3416020 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

The system shall determine whether the user's mobile device is within the MAX_CONNECT_RANGE when the user request is received

##### 19.1.3 VF_PF_SYSRS_1676

| CodeBeamer reference: | APAVPCSR-3416021 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

The system shall determine whether the key fob is within the MAX_CONNECT_RANGE when the user request is received

##### 19.1.4 VF_PF_SYSRS_1677

| CodeBeamer reference: | APAVPCSR-3416022 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall display messages in MHU to keep the driver updated on relevant status in INFO_DISPLAY_TIME (100 milliseconds)

##### 19.1.5 VF_PF_SYSRS_1678

| CodeBeamer reference: | APAVPCSR-3416023 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

APA system shall update the APA status within APA_STS_RESP_TIME

##### 19.1.6 VF_PF_SYSRS_1679

| CodeBeamer reference: | APAVPCSR-3416024 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

The system shall process and record telematics data with a latency of no more than 2 seconds

##### 19.1.7 VF_PF_SYSRS_1680

| CodeBeamer reference: | APAVPCSR-3416025 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

The V-Parking Comfort shall display the detected parking slot on MHU within the SLOT_DISPLAY_TIME sec.

##### 19.1.8 VF_PF_SYSRS_1681

| CodeBeamer reference: | APAVPCSR-3416026 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall display messages in C-app to keep the driver updated on relevant status.

##### 19.1.9 VF_PF_SYSRS_1682

| CodeBeamer reference: | APAVPCSR-3416027 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall display messages in C-app to keep the driver updated on relevant status within CAPP_STS_RESP_TIME <TBD>, rephrase

##### 19.1.10 VF_PF_SYSRS_1683

| CodeBeamer reference: | APAVPCSR-3416028 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

VPC system shall update the VPC status within VPC_STS_RESP_TIME

##### 19.1.11 VF_PF_SYSRS_1684

| CodeBeamer reference: | APAVPCSR-3416029 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

VPC system shall update the the target coordinates to the selected slot coordinates within 500milliseconds to ensure smooth and uninterrupted path planning and execution.

##### 19.1.12 VF_PF_SYSRS_1685

| CodeBeamer reference: | APAVPCSR-3416030 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort module shall compute all feasible trajectories from the current vehicle position to the target parking slot within 100 milliseconds under standard operating conditions.

##### 19.1.13 VF_PF_SYSRS_1686

| CodeBeamer reference: | APAVPCSR-3416031 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort module shall set the Nose-in and Nose- out feasibility within 100milliseconds

##### 19.1.14 VF_PF_SYSRS_1687

| CodeBeamer reference: | APAVPCSR-3416032 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

The V-Parking Comfort shall display the detected parking slot on C-app within the SLOT_DISPLAY_TIME sec.

#### 19.2 Ergonomic Requirements

| CodeBeamer reference: | APAVPCSR-3415746 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 19.2.1 --

| CodeBeamer reference: | APAVPCSR-3416033 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

NA

#### 19.3 Environmental Requirements

| CodeBeamer reference: | APAVPCSR-3415747 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 19.3.1 --

| CodeBeamer reference: | APAVPCSR-3416034 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| AVPCSR-3416600 | -- |
| --- | --- |

No Downstream References

No Associations

Description

The V-Parking Comfort shall operate in the following conditions:
-Extreme Sub-zero ambient temperatures (Ambient temperature < -20 )
-Moderate Sub-zero ambient temperatures ( -20 <= Ambient temperature <= 0)
- Moderate ambient temperatures (0<Ambient temperature<=45)
-High ambient temperatures ( Ambient temperature > 45 °C)

##### 19.3.2 --

| CodeBeamer reference: | APAVPCSR-3416035 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| AVPCSR-3416600 | -- |
| --- | --- |

No Downstream References

No Associations

Description

The V-Parking Comfort shall operate in the following conditions:
- Sunny
- Light Rain
- Moderate Rain

##### 19.3.3 --

| CodeBeamer reference: | APAVPCSR-3416036 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| AVPCSR-3416600 | -- |
| --- | --- |

No Downstream References

No Associations

Description

The V-Parking Comfort shall operate in the following conditions:
- Drizzle
<0.8 Raindrop diameter mm or 0.5~0.9 Rainfall l/m2/h or 0~3 Raindrop Speed (m/s)
- Light rain
<4 Raindrop diameter mm or 2~4 Rainfall l/m2/h or 2~5 Raindrop Speed (m/s)
- Moderate rain
<6 Raindrop diameter mm or 8~12 Raindrop Speed (m/s) or 4~6 Raindrop Speed (m/s)

##### 19.3.4 --

| CodeBeamer reference: | APAVPCSR-3416037 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| AVPCSR-3416638 | -- |
| --- | --- |
| AVPCSR-3416600 | -- |

No Downstream References

No Associations

Description

The V-Parking Comfort shall classify illumination based on sensor data and operate in the following conditions:

- Good Illumination Between 100 - 100000 lx Lowest level is: Very dark overcast day
- Moderate Illumination Between 5 and 100 lx Examples for 10 lx are twilight, street lighting
- Poor Illumination Between 1 and 5 lx Lowest level is Deep Twilight

##### 19.3.5 --

| CodeBeamer reference: | APAVPCSR-3416038 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| AVPCSR-3416600 | -- |
| --- | --- |

No Downstream References

No Associations

Description

The V-Parking Comfort shall classify the visibility range based on sensor data to operate in the following conditions:
- Good Visibility (during rainfall / snowfall / fog / smoke / smog / sleet) more than 300m
- Moderate Visibility (during rainfall / snowfall / fog / smoke / smog /sleet) between 75 and 300m
- Poor Visibility (during rainfall / snowfall / fog / smoke / smog/sleet) between 25 and 75m

##### 19.3.6 --

| CodeBeamer reference: | APAVPCSR-3416039 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| AVPCSR-3416600 | -- |
| --- | --- |

No Downstream References

No Associations

Description

The V-Parking Comfort shall operate in the following conditions based on the sensor data:
- Light wind (Wind speed is lesser than 11 km/h)
- Moderate wind (Wind speed is between 12-28 km/h)

##### 19.3.7 --

| CodeBeamer reference: | APAVPCSR-3416040 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

No Upstream References

No Downstream References

No Associations

Description

The V-Parking Comfort shall function effectively under conditions of electromagnetic interference (EMI)

#### 19.4 Statutory and Regulatory Requirements

| CodeBeamer reference: | APAVPCSR-3415748 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 19.4.1 Descrption

| CodeBeamer reference: | APAVPCSR-3416041 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

V-Comfort complies with regulations and standards, ensuring safe operation, user override capability, and consistent system behavior under all conditions."

##### 19.4.2 Input Signals

| CodeBeamer reference: | APAVPCSR-3416042 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 19.4.3 Output Signals

| CodeBeamer reference: | APAVPCSR-3416043 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 19.4.4 Internal Signals

| CodeBeamer reference: | APAVPCSR-3416044 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 19.4.5 VF_PF_SYSRS_1674

| CodeBeamer reference: | APAVPCSR-3416045 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| AVPCSR-3416559 | -- |
| --- | --- |
| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |

No Downstream References

No Associations

Description

When
VPC_mode is Path_Maneuvering/moving
AND
user shifts the transmission into Park, 
Then 
V-Parking Comfort shall inform the driver and stop the vehicle.

##### 19.4.6 --

| CodeBeamer reference: | APAVPCSR-3416046 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (3)

| APAVPC-3415600 | V_Comfort Park-Out Maneuver Controls |
| --- | --- |
| APAVPC-3415564 | Low-speed forward navigation Execution |
| APAVPC-3415584 | Park-In Maneuver Initiation: Control mode change |

No Downstream References

No Associations

Description

When 
Driver is Operating remotely with key-fob/C-App
Then 
V-Parking comfort shall limit the Vehicle_speed<=10km/h 
Note: Here Vehicle_speed tolerance is 2km/h.

##### 19.4.7 --

| CodeBeamer reference: | APAVPCSR-3416047 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415565 | Remote Mode Selection: Continuous Engagement via Mobile App |
| --- | --- |
| APAVPC-3415561 | Parking Slot Detection |

Downstream References (1)

| APASAD-3412216 | Sequence Diagram |
| --- | --- |

No Associations

Description

V-Parking Comfort shall present the parking space detection notification before the rearmost part of the SV crosses the termination boundary.

##### 19.4.8 --

| CodeBeamer reference: | APAVPCSR-3416048 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415507 | Driver Feedback During V_Comfort Operations |
| --- | --- |
| APAVPC-3415609 | Visual and Auditory Alerts for Driver Communication |

No Downstream References

No Associations

Description

The V-Parking Comfort system shall prevent the Subject Vehicle (SV) from making contact with any pedestrian at any point during its operation.

##### 19.4.9 --

| CodeBeamer reference: | APAVPCSR-3416049 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_Status is Path maneuvering/moving 
AND
Driver press the brake pedal 5% or less of its fully applied position
Then
V-Parking Comfort shall allow manual override by the driver.
Note: The fully applied brake pedal position is defined as the position achieved When a force of 112 lbf (500 N) is applied to the brake pedal.

##### 19.4.10 --

| CodeBeamer reference: | APAVPCSR-3416050 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort system shall comply with all applicable requirements of UN ECE R79 (Steering Equipment), specifically those sections Remote Controlled Parking (RCP), even if the remote control feature itself is not implemented in the system.

##### 19.4.11 --

| CodeBeamer reference: | APAVPCSR-3416051 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (7)

| APAVPC-3415554 | Selection of Control mode for V_Comfort Operation |
| --- | --- |
| APAVPC-3415559 | Pre-Maneuver Condition Check and Status Communication |
| APAVPC-3415543 | V_Comfort Park-In Selection via MHU |
| APAVPC-3415542 | Path and Trajectory Planning: Traffic circulation |
| APAVPC-3415545 | Remote Mode Selection: Occupancy detection |
| APAVPC-3415544 | Communicate activation status to driver |
| APAVPC-3415551 | Reuse |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

V-Parking Comfort system shall be activated after driver press the start button in the preferred communication device(keyfob/C-App) and provided that all necessary operating conditions are met with proper functioning of all associated functions, such as the brakes, accelerator, steering, and relevant sensor systems (e.g., camera, radar, lidar).

##### 19.4.12 --

| CodeBeamer reference: | APAVPCSR-3416052 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

When
VPC_Status is path maneuvering/moving
AND
VPC_cancelled is true
OR
APA_status is APA _disabled 
Then
V-parking Comfort shall set the vehicle stationery, Vehicle speed =0 and engage the parking brake.
Note: Whenever vehicle speed reaches "0 kmph" EPB may or may not engage automatically based on user configuration

##### 19.4.13 --

| CodeBeamer reference: | APAVPCSR-3416053 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415506 | Multi-Interface Support for V_Comfort User Interaction |
| --- | --- |
| APAVPC-3415545 | Remote Mode Selection: Occupancy detection |

Downstream References (1)

| APASAD-3412216 | Sequence Diagram |
| --- | --- |

No Associations

Description

V-Parking Comfort shall support only one type of Parking assist based on the scenarios 
1. Controlled/supervised remotely by a driver who is not necessarily located in the driver's seat

##### 19.4.14 --

| CodeBeamer reference: | APAVPCSR-3416054 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (2)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |
| APASAD-3412216 | Sequence Diagram |

No Associations

Description

When 
The remote driver switch off power in remote device 
Then 
V-parking Comfort shall set VPC_cancelled as true and VPC_status as Abort
AND
inform the driver in C-App

##### 19.4.15 --

| CodeBeamer reference: | APAVPCSR-3416055 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
VPC_mode is Path_Maneuvering/moving
AND
user shifts the transmission into Park, 
Then 
V-Parking Comfort shall set VPC_status as abort and cancel the parking maneuver
AND
informs the driver and stop the vehicle.

##### 19.4.16 --

| CodeBeamer reference: | APAVPCSR-3416056 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When the conventional driver performs other gear shift operations, then the V-Parking Comfort shall stop the vehicle and inform the driver.

##### 19.4.17 --

| CodeBeamer reference: | APAVPCSR-3416057 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When the conventional driver applies braking, then the V-Parking Comfort shall operate according to the amount of braking applied by the driver if it exceeds the V-Parking Comfort-generated braking.

##### 19.4.18 --

| CodeBeamer reference: | APAVPCSR-3416058 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

When the remote driver turns the Ignition OFF, then the V-Parking Comfort shall stop the vehicle and cancel the automatic control of the V-Parking Comfort.

##### 19.4.19 --

| CodeBeamer reference: | APAVPCSR-3416059 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (2)

| APAVPC-3415571 | Parking Feasibility Check |
| --- | --- |
| APAVPC-3415492 | Slot Detection Capability |

Downstream References (1)

| APASAD-3412216 | Sequence Diagram |
| --- | --- |

No Associations

Description

When the parking maneuver is performed within a parallel parking space defined by two bordering vehicles and optionally a kerb as a lateral reference, then the V-Parking Comfort shall calculate the parking space width (W) and depth (D) based on the vehicle's length and a correction factor (Δy) and shall be capable of detecting the kerb present.

##### 19.4.20 --

| CodeBeamer reference: | APAVPCSR-3416060 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415492 | Slot Detection Capability |
| --- | --- |

No Downstream References

No Associations

Description

When the APS vehicle length is between 4m and 6m, then the V-Parking Comfort shall set Δy as 0.25 times the APS vehicle length.

##### 19.4.21 --

| CodeBeamer reference: | APAVPCSR-3416061 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415492 | Slot Detection Capability |
| --- | --- |

No Downstream References

No Associations

Description

When the APS vehicle length is <= 4m, then the V-Parking Comfort shall set Δy to 1.0m (4m x 0.25).

##### 19.4.22 --

| CodeBeamer reference: | APAVPCSR-3416062 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415492 | Slot Detection Capability |
| --- | --- |

No Downstream References

No Associations

Description

When the APS vehicle length is >= 6m, then the V-Parking Comfortshall set Δy to 1.5m (6m x 0.25).

##### 19.4.23 --

| CodeBeamer reference: | APAVPCSR-3416063 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

When the APS-controlled vehicle is in motion, then the V-Parking Comfort shall ensure it remains within the area where APS control is permitted.

##### 19.4.24 --

| CodeBeamer reference: | APAVPCSR-3416064 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

When the parking maneuver is performed within a parking slot defined by contrastive markings on the ground surface, then the V-Parking Comfort shall ensure that the markings have a minimum contrast ratio of 5 and that the APA-controlled vehicle remains within the area where PAPS control is permitted, as shown in the below figure :

##### 19.4.25 --

| CodeBeamer reference: | APAVPCSR-3416065 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

When the parking or leaving maneuver is performed within a parking space defined by two bordering vehicles, then the V-Parking Comfort shall ensure that the parking space width (W) is the width of the PAPS vehicle, including side-view mirrors, plus Δx (=1.2 m), and the PAPS-controlled vehicle shall remain within the area where PAPS control is permitted, as indicated in the figure below:

##### 19.4.26 --

| CodeBeamer reference: | APAVPCSR-3416066 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

When the parking or leaving manoeurve is performed within a perpendicular parking slot, then the V-Parking Comfort shall ensure that the slot is defined by constrastive markings on the ground surface with a minimum contrast ratio of 5, and the APS controlled vehicle shall remain within the area ehere PAPS control is permitted, as shown in the below figure:

##### 19.4.27 --

| CodeBeamer reference: | APAVPCSR-3416067 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

When the parking maneuver is in progress, then the V-Parking Comfort shall advance to the next operating state only after the driver has completed the specified action.

##### 19.4.28 --

| CodeBeamer reference: | APAVPCSR-3416068 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

When the system is in the process of leaving the maneuver, then the V-Parking Comfort shall advance the next operating state only after the driver has completed the described activity.

##### 19.4.29 --

| CodeBeamer reference: | APAVPCSR-3416069 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415623 | Abort Conditions During Automatic Maneuvering |
| --- | --- |

No Downstream References

No Associations

Description

When
The vehicle exceeds the speed limit for the assisted parking mode. 
Then
V-Parking Comfort shall set VPC_Status as abort during assisted parking mode

#### 19.5 Audible Information:

| CodeBeamer reference: | APAVPCSR-3415749 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 19.5.1 Descrption

| CodeBeamer reference: | APAVPCSR-3416070 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 19.5.2 --

| CodeBeamer reference: | APAVPCSR-3416071 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415507 | Driver Feedback During V_Comfort Operations |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

When presenting audible information, then the system shall ensure it is in accordance with ISO 15006.

##### 19.5.3 --

| CodeBeamer reference: | APAVPCSR-3416072 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415507 | Driver Feedback During V_Comfort Operations |
| --- | --- |

No Downstream References

No Associations

Description

When encoding distance information, then the system shall use at least two levels with different repetition rates, where a higher repetition rate or continuous sound corresponds to shorter distances.

##### 19.5.4 --

| CodeBeamer reference: | APAVPCSR-3416073 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415507 | Driver Feedback During V_Comfort Operations |
| --- | --- |

No Downstream References

No Associations

Description

When distinguishing between areas, then the system shall use a maximum of two different carrier frequencies (e.g., high frequency for the front and low frequency for the rear).

##### 19.5.5 --

| CodeBeamer reference: | APAVPCSR-3416074 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415507 | Driver Feedback During V_Comfort Operations |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

When providing activation, deactivation, or failure indications, then the system shall use clearly distinguishable audible signals.

#### 19.6 Visual Information:

| CodeBeamer reference: | APAVPCSR-3415750 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 19.6.1 --

| CodeBeamer reference: | APAVPCSR-3416075 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415507 | Driver Feedback During V_Comfort Operations |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

When presenting visual information, then the system shall ensure it is in accordance with ISO 15008.

##### 19.6.2 --

| CodeBeamer reference: | APAVPCSR-3416076 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415507 | Driver Feedback During V_Comfort Operations |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

When using color-coded levels, then the system shall use red for level 1 (imminent collision level) and yellow/green for level 2 (attention level).

##### 19.6.3 --

| CodeBeamer reference: | APAVPCSR-3416077 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415507 | Driver Feedback During V_Comfort Operations |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

When indicating activation, deactivation, and malfunction, then the system shall use a telltale or a symbol in accordance with ISO 2575

##### 19.6.4 Descrption

| CodeBeamer reference: | APAVPCSR-3416078 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 19.7 Startup detection delay

| CodeBeamer reference: | APAVPCSR-3415751 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 19.7.1 Descrption

| CodeBeamer reference: | APAVPCSR-3416079 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 19.7.2 --

| CodeBeamer reference: | APAVPCSR-3416080 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

No Upstream References

No Downstream References

No Associations

Description

When the system is activated, then it shall present correct obstacle information within 1.5 seconds, including time for internal system and sensor tests.

##### 19.7.3 --

| CodeBeamer reference: | APAVPCSR-3416081 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

No Upstream References

No Downstream References

No Associations

Description

When no readiness-for-service indication is provided, then the delay shall be measured from when the ignition is ON and the engine is running, or when battery voltage reaches 90% of its stationary level.

##### 19.7.4 --

| CodeBeamer reference: | APAVPCSR-3416082 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

No Upstream References

No Downstream References

No Associations

Description

When an audible or visual readiness-for-service indication is provided, then the start-up detection delay shall be measured from when the indication ends and shall not exceed an average of 600ms.

#### 19.8 Monitoring range coverage

| CodeBeamer reference: | APAVPCSR-3415752 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 19.8.1 Descrption

| CodeBeamer reference: | APAVPCSR-3416083 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 19.9 Detection and Response Time:

| CodeBeamer reference: | APAVPCSR-3415753 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 19.9.1 Descrption

| CodeBeamer reference: | APAVPCSR-3416084 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 19.9.2 --

| CodeBeamer reference: | APAVPCSR-3416085 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

The system shall codify distance indications into at least two zones. These zones may be represented by different repetition rates or variable frequencies, with the basic rule that a high repetition rate, continuous sound, or high frequency corresponds to short distances.

##### 19.9.3 --

| CodeBeamer reference: | APAVPCSR-3416086 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

No Upstream References

No Downstream References

No Associations

Description

When detecting targets, then the system shall detect targets located in the 1.0 m to 5.0 m range behind the vehicle's rear bumper in a region called the zone of regard.

##### 19.9.4 --

| CodeBeamer reference: | APAVPCSR-3416087 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

No Upstream References

No Downstream References

No Associations

Description

When measuring response time, then the system shall maintain an average response time of no more than 450 milliseconds, ensuring that no individual measurement exceeds 550 milliseconds for systems with no visual or audible readiness-for-service indication.

##### 19.9.5 --

| CodeBeamer reference: | APAVPCSR-3416088 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

No Upstream References

No Downstream References

No Associations

Description

When the system has only a visual readiness-for-service indication, then the average response time shall not exceed 150 ms, with no single measurement value exceeding 250 ms.

##### 19.9.6 --

| CodeBeamer reference: | APAVPCSR-3416089 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

No Upstream References

No Downstream References

No Associations

Description

When the system has only an audible readiness-for-service indication, then the increased response time shall allow for a period of silence between the audible sound of the readiness-for-service tone and the audible sound of the azimuth zone of regard.

##### 19.9.7 --

| CodeBeamer reference: | APAVPCSR-3416090 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

No Upstream References

No Downstream References

No Associations

Description

When the system is active, then the average time delay between the appearance of a relevant obstacle in the zone of regard and the presentation of indications must not exceed 150 milliseconds.
When measuring time delay, then no single measurement value should exceed 250 milliseconds.

#### 19.10 Description of the azimuth zone of regard - Presence

| CodeBeamer reference: | APAVPCSR-3415754 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 19.10.1 --

| CodeBeamer reference: | APAVPCSR-3416091 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| AVPCSR-3416688 | -- |
| --- | --- |

No Downstream References

No Associations

Description

The APA System shall define the azimuth area composed of eight smaller zones defined as follows: 
a) Bnear zone: a near backing zone centred on the vehicle's longitudinal centreline. It covers 80 % of the bumper width, extending longitudinally from 1,0 m rearward of the bumper to at least 4,0 m rearward of the bumper, as shown in Figure 2 
b) Bedge zones: two backing edge zones located adjacent to the Bnear and Bfar zones. Each Bedge zone begins at the edge of the Bnear and Bfar zones and extends laterally to 0,25 m past the rear bumper side edge as shown in Figure 1. Each Bedge zone extends longitudinally from 1,0 m rearward of the bumper to at least 5,0 m rearward of the bumper. 
c) Bfar zone: a far backing zone aft of the Bnear zone centred on the vehicle longitudinal centreline. It covers 80 % of the of the bumper width, extending longitudinally from at least 4,0 m (80 % of the required 5,0 m minimum backing range) to at least 5,0 m (100 % of the required 5,0 m minimum backing range) as shown in Figure 2 
d) Bside zones: two backing side zones located adjacent to the Bedge zones. The purpose of the Bside zones is to restrict the detection of Objects (e.g. parallel-parked vehicles) in this region in order to minimize nuisance warnings. Each Bside zone begins at the edge of the Bedge zone and extends laterally 0,25 m beyond the rear bumper lateral (side) edge as shown in Figure 1. Each Bside zone extends longitudinally from 1,0 m rearward of the bumper to at least 5,0 m rearward of the bumper (100 % of the required 5,0 m minimum backing range). 
e) Bout zones: two backing out zones located adjacent to the Bside zones. The purpose of the Bout zones is to restrict the detection of Objects far outside the path of the subject vehicle (e.g. adjacent-lane vehicles) in order to minimize nuisance warnings. Each Bout zone begins at the edge of the Bside zone and extends laterally for 1,0 m as shown in Figure 1. Each Bout zone extends longitudinally from 1,0 m rearward of the bumper to at least 5,0 m rearward of the bumper (100 % of the required 5,0 m minimum backing range).

##### 19.10.2 --

| CodeBeamer reference: | APAVPCSR-3416092 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

##### 19.10.3 --

| CodeBeamer reference: | APAVPCSR-3416093 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

Here Key terms: 

Bnear: Backing area, near 

Bedge : Backing area, edge 

Bfar: Backing area, far 

Bside: Backing area, side 

Bout: Backing area, out 



Dimensions areas: 

A = 1.0 m 

B = 0.25 m 

C =detection range 

D = bumper width 

a = 10 cm × 10 cm. 

b = 0.1 m to 0.25 m beyond. 

[!dd80338bf905205a1b5fe8c0c316a864.tmp!]

##### 19.10.4 --

| CodeBeamer reference: | APAVPCSR-3416094 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

#### 19.11 Maintenance/ Support Requirements

| CodeBeamer reference: | APAVPCSR-3415755 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 19.11.1 --

| CodeBeamer reference: | APAVPCSR-3416095 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

TBD

#### 19.12 Quality Requirements

| CodeBeamer reference: | APAVPCSR-3415756 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 19.12.1 Descrption

| CodeBeamer reference: | APAVPCSR-3416096 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 19.12.2 --

| CodeBeamer reference: | APAVPCSR-3416097 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415534 | Adherence to ASPICE standards for Quality Processes |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

V-Parking Comfort shall adhere to the ASPICE standards for quality processes.

##### 19.12.3 --

| CodeBeamer reference: | APAVPCSR-3416098 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Released |

Upstream References (1)

| APAVPC-3415533 | Adherence to INCOSE standards for Systems Engineering. |
| --- | --- |

Downstream References (1)

| APASAD-3412218 | Usecase Diagram |
| --- | --- |

No Associations

Description

V-Parking Comfort shall adhere to INCOSE standards for Systems Engineering.

#### 19.13 EMI-EMC requirements

| CodeBeamer reference: | APAVPCSR-3415757 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 19.14 Sensor Requirement

| CodeBeamer reference: | APAVPCSR-3415758 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 19.15 Actuators Requirement

| CodeBeamer reference: | APAVPCSR-3415759 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 19.16 End of Line Requirements

| CodeBeamer reference: | APAVPCSR-3415760 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 19.17 Re-programming requirements

| CodeBeamer reference: | APAVPCSR-3415761 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 19.18 Exception Handling

| CodeBeamer reference: | APAVPCSR-3415762 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 19.18.1 --

| CodeBeamer reference: | APAVPCSR-3416099 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

TBD

#### 19.19 Response to Erroneous Conditions

| CodeBeamer reference: | APAVPCSR-3415763 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 19.20 Recovery From Errors

| CodeBeamer reference: | APAVPCSR-3415764 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 19.21 Diagnostic Requirements

| CodeBeamer reference: | APAVPCSR-3415765 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 19.21.1 UDS

| CodeBeamer reference: | APAVPCSR-3416100 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

###### 19.21.1.1 VF_PF_SYSRS_1688

| CodeBeamer reference: | APAVPCSR-3416299 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | Ready for QA Review |

No Upstream References

No Downstream References

No Associations

Description

V-Parking Comfort shall enable following parameters access via UDS Services 0x22(Read Data) and 0x2E(Write Data)
Obstcl_Identified
Sensor_Malfunction
Moving_Control_Timeout
Trajectory_deviation_detected
Target_Coordinate
Nose-in_feasible
Nose-out_feasible
Park_type
User_Comm_dev
Slot_Detected
Alternate_Path_Planning
Movable_object_in_path
Note: Core UDS Services has to be implemented as part of ADECU stack.

#### 19.22 Requirements on the test interface

| CodeBeamer reference: | APAVPCSR-3415766 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 19.23 Electrical Requirements

| CodeBeamer reference: | APAVPCSR-3415767 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 19.24 Electrical Characteristics

| CodeBeamer reference: | APAVPCSR-3415768 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 19.25 Hardware Performance Characteristics

| CodeBeamer reference: | APAVPCSR-3415769 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 19.26 Mechanical Requirements

| CodeBeamer reference: | APAVPCSR-3415770 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 19.27 Mechanical characteristics

| CodeBeamer reference: | APAVPCSR-3415771 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 19.28 Physical Requirements

| CodeBeamer reference: | APAVPCSR-3415772 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 19.29 Packaging requirements

| CodeBeamer reference: | APAVPCSR-3415773 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 19.30 Manual Operations Requirements

| CodeBeamer reference: | APAVPCSR-3415774 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

##### 19.30.1 --

| CodeBeamer reference: | APAVPCSR-3416101 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

TBD

#### 19.31 Mechanical Operation requirements

| CodeBeamer reference: | APAVPCSR-3415775 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 19.32 Assembly Requirements

| CodeBeamer reference: | APAVPCSR-3415776 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 19.33 Installation Requirement

| CodeBeamer reference: | APAVPCSR-3415777 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 19.34 Hardware Platform

| CodeBeamer reference: | APAVPCSR-3415778 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 19.35 Business/ Organizational Requirements

| CodeBeamer reference: | APAVPCSR-3415779 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 19.36 Reusable system elements

| CodeBeamer reference: | APAVPCSR-3415780 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

### 20 References

| CodeBeamer reference: | APAVPCSR-3415654 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

--

#### 20.1 --

| CodeBeamer reference: | APAVPCSR-3415781 |
| --- | --- |
| Revision: | 4 |
| Priority: | -- |
| Severity: | -- |
| Status: | -- |

No Upstream References

No Downstream References

No Associations

Description

TE_VF_ParkingFunction_Stakeholder_Req_V-Parking_Comfort.xlsx version: 2.0